Chapter 3 Exercise

### 1. What do the following MOV instructions accomplish?

(a) MOV AX, BX → Copies contents of BX into AX.
(b) MOV BX, AX → Copies contents of AX into BX.
(c) MOV BL, CH → Copies contents of CH into BL.
(d) MOV ESP, EBP → Copies contents of EBP into ESP.
(e) MOV RAX, RCX → Copies contents of RCX into RAX.

### 2. List the 8-bit registers that are used for register addressing.

Answer: AL, BL, CL, DL, AH, BH, CH, DH.

### 3. List the 16-bit registers that are used for register addressing.

Answer: AX, BX, CX, DX, SI, DI, BP, SP.

### 4. List the 32-bit registers that are used for register addressing in the 80386 through the Core2 microprocessors.

Answer: EAX, EBX, ECX, EDX, ESI, EDI, EBP, ESP.

### 5. List the 64-bit registers available to the 64-bit mode of the Pentium 4 and Core2.

Answer: RAX, RBX, RCX, RDX, RSI, RDI, RBP, RSP, R8–R15.

### 6. List the 16-bit segment registers used with register addressing by MOV, PUSH, and POP.

Answer: CS (Code Segment), DS (Data Segment), ES (Extra Segment), SS (Stack Segment), FS, GS.

### 7. What is wrong with the MOV BL, CX instruction?(EXAM)

Answer: Operand sizes don’t match — BL is 8-bit, CX is 16-bit.

### 8. What is wrong with the MOV DS, SS instruction?(EXAM)

Answer: You cannot directly move data between segment registers. You must use a general-purpose register as an intermediate.

### 9. Select an instruction for each of the following tasks:

(a) Copy EBX into EDX → MOV EDX, EBX
(b) Copy BL into CL → MOV CL, BL
(c) Copy SI into BX → MOV BX, SI
(d) Copy DS into AX → MOV AX, DS
(e) Copy AL into AH → MOV AH, AL
(f) Copy R8 into R10 → MOV R10, R8

### 10. Select an instruction for each of the following tasks:

(a) Move 12H into AL → MOV AL, 12H
(b) Move 123AH into AX → MOV AX, 123AH
(c) Move 0CDH into CL → MOV CL, 0CDH
(d) Move 1000H into RAX → MOV RAX, 1000H
(e) Move 1200A2H into EBX → MOV EBX, 1200A2H

### 11. What special symbol is sometimes used to denote immediate data?

Answer: The # (hash) symbol (e.g., MOV AL, #25H).

### 12. What is the purpose of the .MODEL TINY statement?

Answer: Defines a memory model where code and data share the same segment (single segment model).

### 13. What assembly language directive indicates the start of the CODE segment?(EXAM)

Answer: .CODE

### 14. What is a label?

Answer: A name used to identify a memory location or instruction address, e.g.,
START: MOV AX, BX.

### 15. The MOV instruction is placed in what field of a statement?

Answer: In the opcode (mnemonic) field of the assembly statement.

### 16. A label may begin with what characters?

Answer:
A label may begin with a letter (A–Z or a–z) or underscore (_).
It cannot begin with a digit.

### 17. What is the purpose of the .EXIT directive?

Answer:
The .EXIT directive tells the assembler to return control to the operating system — it marks the end of the program execution.

### 18. Does the .MODEL TINY statement cause a program to assemble as an .EXE program?

Answer:
No. The .MODEL TINY directive assembles a .COM (command) type program,
not an .EXE file.
(COM programs have code and data in one segment.)

### 19. What tasks does the .STARTUP directive accomplish in the small memory model?

Answer:
The .STARTUP directive:

- Initializes the DS (Data Segment) register.

- Sets up the stack segment automatically.

- Jumps to the program’s starting point for execution.

(Used in small memory model programs to handle segment setup.)

### 20. What is a displacement? How does it determine the memory address in a MOV DS:[2000H], AL instruction?(EXAM)

Answer:

- A displacement is the offset value (the distance from the start of a segment).

- In MOV DS:[2000H], AL,
the effective address (EA) = 2000H.

- The physical address = (DS × 10H) + 2000H.

If DS = 1000H, → Physical address = 10000H + 2000H = 12000H.

### 21. What do the symbols [ ] indicate?

Answer:
Brackets [ ] indicate memory addressing —
that the operand refers to a memory location, not a register or immediate data.
Example:
MOV AL, [BX] → Move the byte from memory at address in BX into AL.

### 22. Suppose that DS = 0200H, BX = 0300H, and DI = 0400H. Determine the memory address accessed by each instruction (real mode).

#### (a) MOV AL, [1234H]

Effective address = 1234H
Physical address = (DS × 10H) + 1234H = (0200 × 10H) + 1234H = 3234H

#### (b) MOV EAX, [BX]

Effective address = BX = 0300H
Physical address = (DS × 10H) + 0300H = 02000H + 0300H = 02300H

#### (c) MOV [DI], AL

Effective address = DI = 0400H
Physical address = (DS × 10H) + 0400H = 02000H + 0400H = 02400H

### 23. What is wrong with a MOV [BX], [DI] instruction?

Answer:
Both operands are memory operands, but the MOV instruction does not allow memory-to-memory transfer directly.
→ You must use a register as an intermediate.
Example:

MOV AL, [BX]

MOV [DI], AL

### 24. Choose an instruction that requires BYTE PTR.

Answer:
MOV BYTE PTR [SI], AL
(Specifies that the operand is a byte in memory.)

### 25. Choose an instruction that requires WORD PTR.

Answer:
MOV WORD PTR [BX], AX
(Specifies that the operand is a 16-bit word in memory.)

### 26. Choose an instruction that requires DWORD PTR.

Answer:
MOV DWORD PTR [DI], EAX
(Specifies that the operand is a 32-bit double word in memory.)

### 27. Select an instruction that requires QWORD PTR.

Answer:
MOVQ QWORD PTR [RAX], XMM0
(Used in 64-bit mode for 8-byte memory operations.)

### 28. Explain the difference between the MOV BX, DATA instruction and the MOV BX, OFFSET DATA instruction.

| Instruction | Meaning | Result |
| --- | --- | --- |
| MOV BX, DATA | Moves the contents (value) stored at label DATA | BX ← [DATA] |
| MOV BX, OFFSET DATA | Moves the address (offset) of label DATA | BX ← Address of DATA |

### 29. Suppose that DS = 1000H, SS = 2000H, BP = 1000H, and DI = 0100H. Determine the memory address accessed by each instruction (real mode).

#### (a) MOV AL, [BP+DI]

Uses SS by default (because BP is used).
EA = BP + DI = 1000H + 0100H = 1100H
Physical = (SS × 10H) + 1100H = (2000H × 10H) + 1100H = 21100H

#### (b) MOV CX, [DI]

Uses DS by default.
EA = DI = 0100H
Physical = (DS × 10H) + 0100H = (1000H × 10H) + 0100H = 10100H

#### (c) MOV EDX, [BP]

Uses SS by default.
EA = BP = 1000H
Physical = (SS × 10H) + 1000H = (2000H × 10H) + 1000H = 21000H

### 30. What, if anything, is wrong with a MOV AL, [BX][SI] instruction?

Answer:
✅ Nothing is wrong — it is valid.
[BX][SI] means BX + SI addressing mode.
Equivalent to:
MOV AL, [BX + SI]