Chapter 04

### 1. The first byte of an instruction is the ____________, unless it contains one of the override prefixes.

✅ Answer: The opcode.
🧠 Explanation: The opcode specifies the operation the processor will perform.

### 2. Describe the purpose of the D- and W-bits found in some machine language instructions.

✅ Answer:

- D-bit (Direction bit): Determines the direction of data transfer — whether data moves from register to memory or from memory to register.

- W-bit (Width bit): Indicates the size of data — 0 for 8-bit and 1 for 16-bit (or 32-bit) operations.

### 3. In a machine language instruction, what information is specified by the MOD field?

✅ Answer:
The MOD field specifies the addressing mode — whether the operand is a register, a memory address, or a memory address with displacement.

### 4. If the register field (REG) of an instruction contains 010 and W = 0, what register is selected, assuming that the instruction is a 16-bit mode instruction?

✅ Answer:
Register selected → DL (8-bit register).
🧠 Because when W = 0, it means 8-bit, and REG = 010 corresponds to DL.

### 5. How are the 32-bit registers selected for the Pentium 4 microprocessor?

✅ Answer:
By using the operand-size prefix (66H) or operating in 32-bit mode, which selects 32-bit registers (EAX, EBX, ECX, etc.) instead of 16-bit registers.

### 6. What memory-addressing mode is specified by R/M = 001 with MOD = 00 for a 16-bit instruction?

✅ Answer:
Addressing mode → [BX + DI]
🧠 Because MOD = 00 and R/M = 001 corresponds to the BX + DI effective address in 16-bit mode.

### 7. Identify the default segment registers assigned to the following:

| Register | Default Segment Register |
| --- | --- |
| (a) SP | SS (Stack Segment) |
| (b) EBX | DS (Data Segment) |
| (c) DI | DS (Data Segment) |
| (d) EBP | SS (Stack Segment) |
| (e) SI | DS (Data Segment) |

### 8. Convert 8B07H from machine language to assembly language.

✅ Answer:
MOV AL, [BX]
🧠 Opcode 8B = MOV, with addressing mode using BX as the base register.

### 9. Convert 8B9E004CH from machine language to assembly language.

✅ Answer:
MOV BX, [SI+4C00H]
🧠 Opcode 8B = MOV, and displacement 4C00H is added to SI.

### 10. If a MOV SI,[BX+2] instruction appears in a program, what is its machine language equivalent?

✅ Answer:
8B 70 02
🧠 Opcode 8B (MOV r16, r/m16), ModR/M byte encodes [BX+2], and 02 is the displacement.

### 11. If a MOV ESI,[EAX] instruction appears in a program for the Core2 microprocessor operating in the 16-bit instruction mode, what is its machine language equivalent?

✅ Answer:
67 8B 30
🧠 Prefix 67H switches addressing mode, 8B = MOV, and ModR/M encodes ESI ← [EAX].

### 12. What is the purpose of REX?

✅ Answer:
The REX prefix is used in 64-bit mode to enable access to extended registers (R8–R15) and specify 64-bit operand size.

### 13. What is wrong with a MOV CS,AX instruction?

✅ Answer:
You cannot move data directly into the Code Segment (CS) using MOV.
🧠 CS can only be changed using FAR JMP, FAR CALL, or RET.

### 14. Form a short sequence of instructions that load the data segment register with 1000H.

✅ Answer:

MOV AX, 1000H

MOV DS, AX

🧠 You must first load the value into AX, then move AX into DS.

### 15. The PUSH and POP instructions always transfer a(n) ____________-bit number between the stack and a register or memory location in the 80386–Core2 microprocessors when operated in the 32-bit mode.

✅ Answer: 32-bit number.

### 16. Create an instruction that places RAX onto the stack in the 64-bit mode for the Pentium 4.

✅ Answer:

PUSH RAX

### 17. What segment register may not be popped from the stack?

✅ Answer: CS (Code Segment)

### 18. Which registers move onto the stack with the PUSHA instruction?

✅ Answer:
AX, CX, DX, BX, SP, BP, SI, DI (in that order).

### 19. Which registers move onto the stack for a PUSHAD instruction?

✅ Answer:
EAX, ECX, EDX, EBX, ESP, EBP, ESI, EDI

### 20. Describe the operation of each of the following instructions:

| Instruction | Description |
| --- | --- |
| (a) PUSH AX | Pushes the contents of AX onto the stack. |
| (b) POP ESI | Pops the top of the stack into ESI. |
| (c) PUSH [BX] | Pushes the data from the memory location pointed to by BX onto the stack. |
| (d) PUSHFD | Pushes the EFLAGS register onto the stack. |
| (e) POP DS | Pops the top of the stack into DS (data segment). |
| (f) PUSHD 4 | Pushes the 32-bit immediate value 4 onto the stack. |

### 21. Explain what happens when the PUSH BX instruction executes. Make sure to show where BH and BL are stored. (Assume SP = 0100H and SS = 0200H.) (Exam)

✅ Answer:

- The PUSH BX instruction stores the contents of BX onto the stack.

- SP is decreased by 2 (because BX is 16-bit).

- BH and BL are stored as follows:

  - Then, BH is stored at [SS:00FFH] and BL at [SS:00FEH]
🧠 Physical address = SS × 10H + SP = 0200H × 10H + 0100H = 02000H + 0100H = 02100H.

### 22. Repeat question 21 for the PUSH EAX instruction.

✅ Answer:

- PUSH EAX pushes the 32-bit contents of EAX onto the stack.

- SP (or ESP in 32-bit mode) is decreased by 4.

- The bytes are stored in order:

  - [SS:SP] ← AL

  - [SS:SP+1] ← AH

  - [SS:SP+2] ← upper bytes of EAX

  - [SS:SP+3] ← highest byte of EAX

### 23. The 16-bit POP instruction (except for POPA) increments SP by ____________.

✅ Answer: 2 bytes

### 24. What values appear in SP and SS if the stack is addressed at memory location 02200H?

✅ Answer:
If stack base = 02200H,
then SS = 0220H and SP = 0000H
🧠 So the stack begins at physical address 02200H × 10H + 0000H = 22000H.

### 25. Compare the operation of a MOV DI,NUMB instruction with an LEA DI,NUMB instruction.

✅ Answer:

- MOV DI, NUMB loads the contents (value) stored at label NUMB into DI.

- LEA DI, NUMB loads the address (offset) of NUMB into DI.

🧠 MOV → gets the data.
LEA → gets the address.

### 26. What is the difference between an LEA SI,NUMB instruction and a MOV SI,OFFSET NUMB instruction?

✅ Answer:
There is no operational difference — both load the offset address of NUMB into SI.
However, LEA can also calculate addresses with registers (e.g. [BX+DI+5]), while OFFSET works only with labels.

### 27. Which is more efficient, a MOV with an OFFSET or an LEA instruction?

✅ Answer:
MOV with OFFSET is more efficient — it uses fewer CPU cycles because the assembler directly substitutes the address value.

### 28. Describe how the LDS BX,NUMB instruction operates.

✅ Answer:
LDS (Load Pointer using DS) loads:

- The offset part of NUMB into BX

- The segment part (next word) into DS

🧠 So LDS loads both offset and data segment address.

### 29. What is the difference between the LDS and LSS instructions?

✅ Answer:

- LDS loads destination register and DS (Data Segment).

- LSS loads destination register and SS (Stack Segment).

### 30. Develop a sequence of instructions that moves the contents of data segment memory locations NUMB and NUMB+1 into BX, DX, and SI.

✅ Answer:

MOV BX, [NUMB]

MOV DX, [NUMB+1]

MOV SI, [NUMB]

### 31. What is the purpose of the direction flag?

✅ Answer:
The Direction Flag (DF) controls the direction of string operations:

- DF = 0 → Process strings forward (increment SI/DI).

- DF = 1 → Process strings backward (decrement SI/DI).

### 32. Which instructions set and clear the direction flag?

✅ Answer:

- STD → Set Direction Flag (DF = 1)

- CLD → Clear Direction Flag (DF = 0)

### 33. Which string instruction(s) use both DI and SI to address memory data?

✅ Answer:
MOVS, CMPS, and SCAS use both DI (Destination Index) and SI (Source Index).

### 34. Explain the operation of the LODSB instruction.

✅ Answer:

- LODSB loads a byte from memory location [DS:SI] into AL.

- After loading:

  - If DF = 0, SI = SI + 1

  - If DF = 1, SI = SI − 1

### 35. Explain the operation of the LODSQ instruction for the 64-bit mode of the Pentium 4 or Core2.

✅ Answer:

- LODSQ loads a 64-bit value from memory [RSI] into RAX.

- Then increments or decrements RSI by 8 bytes depending on the Direction Flag.

### 36. Explain the operation of the OUTSB instruction.

✅ Answer:

- OUTSB outputs a byte from memory location [DS:SI] to the port addressed by DX.

- SI is then incremented or decremented depending on DF.

### 37. Explain the operation of the STOSW instruction.

✅ Answer:

- STOSW stores the word in AX into the memory location [ES:DI].

- Then DI is incremented or decremented by 2 depending on the Direction Flag.

### 38. Develop a sequence of instructions that copy 12 bytes of data from an area of memory addressed by SOURCE into an area of memory addressed by DEST.

✅ Answer:

MOV CX, 12        ; number of bytes to copy

LEA SI, SOURCE    ; source address

LEA DI, DEST      ; destination address

CLD               ; clear direction flag

REP MOVSB         ; repeat move 12 times

### 39. What does the REP prefix accomplish and what type of instruction is it used with?

✅ Answer:

- REP repeats a string instruction until CX (or ECX/RCX) becomes zero.

- Used with string instructions like MOVS, STOS, CMPS, etc.

### 40. Select an assembly language instruction that exchanges the contents of the EBX register with the ESI register.

✅ Answer:

XCHG EBX, ESI.

### 41. Where is the I/O address (port number) stored for an INSB instruction?

Answer: In register DX.
INSB reads a byte from the I/O port whose port number is in DX and stores it at [ES:DI] (then updates DI according to DF).

### 42. Would the LAHF and SAHF instructions normally appear in software?

Answer: Rarely in modern high-level application code.
They are legacy instructions (load/store AH with low byte of FLAGS) used mainly in low-level, legacy, or compatibility code and some device/interrupt handlers. Modern compilers usually avoid them.

### 43. Short program (using XLAT) to convert BCD 0–9 to ASCII (30H–39H) and store ASCII in a TABLE (data segment).

Answer (example 16-bit MASM-style):

; Assume: input BCD digits are in array DIGITS (each 0..9), count in CX

; TABLE contains ASCII '0'..'9' (30H..39H)

MOV BX, OFFSET TABLE    ; BX points to table base (DS:BX)

CONVERT_LOOP:

MOV AL, [SI]            ; get BCD digit (0..9) from source at DS:SI

XLAT                    ; AL := byte ptr [BX + AL]  → ASCII code

MOV [DI], AL            ; store ASCII at DS:DI (destination)

INC SI

INC DI

LOOP CONVERT_LOOP

Data (example):

TABLE DB '0','1','2','3','4','5','6','7','8','9' ; ASCII 30h..39h

### 44. Explain how the XLAT instruction transforms the contents of the AL register.

Answer: XLAT replaces AL with the byte at memory address DS:BX + AL.
Effectively: AL ← [DS : (BX + AL)]. (BX is the table base; AL is used as index.)

### 45. Explain what the IN AL,12H instruction accomplishes.

Answer: Reads a byte from I/O port 12H (immediate 8-bit port) into AL.

### 46. Explain how the OUT DX,AX instruction operates.

Answer: Writes the word (16 bits) in AX to the I/O port whose port number is in DX. (If in 32/64-bit modes, still uses AX as the 16-bit value to send.)

### 47. What is a segment-override prefix?

Answer: A machine-code prefix that tells the processor to use a non-default segment register for the following memory operand (e.g., ES:, CS:, SS:, DS:). Encoded as specific prefix byte(s) before the instruction.

### 48. Select an instruction that moves a byte from memory addressed by BX in the extra segment into AH.

Answer:

MOV AH, ES:[BX]

(or equivalently MOV AH, [ES:BX])

### 49. Develop a sequence that exchanges AX↔BX, ECX↔EDX, SI↔DI.

Answer:

XCHG AX, BX

XCHG ECX, EDX

XCHG SI, DI

### 50. What is an assembly-language directive?

Answer: A statement for the assembler (not executed by CPU) that controls assembly: defines data, reserves storage, sets segments, defines constants, procedure boundaries, etc. (Examples: DB, DW, SEGMENT, PROC, ENDP, EQU.)

### 51. What is accomplished by CMOVNE CX,DX on Pentium 4?

Answer: Conditional move: if ZF = 0 (not equal), move DX → CX; otherwise leave CX unchanged. (No flags are modified.)

### 52. Purpose of directives DB, DW, and DD.

Answer:

- DB — Define Byte (allocate/initialize 8-bit values).

- DW — Define Word (allocate/initialize 16-bit values).

- DD — Define Doubleword (allocate/initialize 32-bit values).

### 53. Directive that reserves 30 bytes for array LIST1.

Answer (MASM):

LIST1 DB 30 DUP (?)

This reserves 30 uninitialized bytes.

### 54. Describe the purpose of the EQU directive.

Answer: name EQU value creates a constant — the assembler substitutes value wherever name appears. (It's a compile-time symbolic constant.)

### 55. What is the purpose of the .686 directive?

Answer: Tells the assembler to enable instructions/features from the 686 (Pentium Pro/Pentium II/III-era) instruction set (e.g., conditional moves). It enables assembler acceptance/encoding of those extended opcodes.

### 56. What is the purpose of the .MODEL directive?

Answer: In MASM it specifies the memory model (tiny, small, medium, compact, large, huge) which affects segment usage and default code/data sizes and how far procedures/data are referenced.

### 57. If the start of a segment is identified with .DATA, what type of memory organization is in effect?(EXAM)

Answer: Data segment — area reserved for program data (variables, arrays, constants).

### 58. If the SEGMENT directive identifies the start of a segment, what type of memory organization is in effect?

Answer: Depends on the segment name used (e.g., CODE SEGMENT → code segment, DATA SEGMENT → data segment, STACK SEGMENT → stack). SEGMENT itself starts whatever named segment follows.

### 59. What does INT 21H accomplish if AH = 4CH?

Answer: DOS terminate program / process: Terminate with return code in AL. Control returns to DOS with AL as exit code.

### 60. What directives indicate the start and end of a procedure?(EXAM)

Answer: PROC (or PROC NEAR / PROC FAR) begins a procedure; ENDP ends it.

### 61. Explain purpose of the USES statement (MASM 6.x) for a procedure.

Answer: USES reglist causes the assembler to emit PUSH instructions for the listed registers at procedure entry and corresponding POP instructions before return — automatically saving/restoring registers for the procedure.

### 62. Develop a near procedure that stores AL in four consecutive data-segment locations addressed by DI.

Answer (MASM 16-bit example):

Store4 PROC NEAR

MOV [DI], AL

MOV [DI+1], AL

MOV [DI+2], AL

MOV [DI+3], AL

RET

Store4 ENDP

### 63. How is the Pentium 4 instructed to use the 16-bit instruction mode?

Answer: Two ways:

- Run the CPU in real mode (native 16-bit instruction decoding).

- In protected/compatibility modes, use the operand-size (66h) and address-size (67h) prefixes to select 16-bit instruction/address encoding for the following instruction(s). (OS also controls the CPU mode.)

### 64. Develop a far procedure that copies the contents of the word-sized memory location CS:DATA4 into AX, BX, CX, DX, SI.

Answer (MASM-style):

CopyFromCS PROC FAR

MOV AX, WORD PTR CS:DATA4   ; load word at CS:DATA4 into AX

MOV BX, AX

MOV CX, AX

MOV DX, AX

MOV SI, AX

RET FAR

CopyFromCS ENDP

If you want, I can:

- Convert these to a compact printable cheat-sheet, or

- Expand any answer with machine-code bytes or a memory diagram for visualization. Which would help you most?