COA Previous solve (19-20)

1(a) Learning outcomes from this course (4 marks)

Ans: এই course থেকে আমি শিখেছি –

- Computer system ভিতরে কীভাবে কাজ করে — CPU, memory, I/O connection।

- Instruction কিভাবে execute হয় এবং data কিভাবে move করে।

- Pipeline, cache, memory management এর concept বুঝেছি।

- ভবিষ্যতে system design, embedded system, এবং software optimization-এ এই knowledge কাজে লাগবে।

👉 In short: এই course আমাকে hardware-software relation এবং efficient system design বুঝতে সাহায্য করেছে।

1(b)(i) What is Computer Family? (1 mark)

একটা Computer Family মানে হলো — একই manufacturer-এর তৈরি এমন কিছু computer যাদের architecture, instruction set একরকম, কিন্তু speed, size বা price আলাদা হতে পারে।

👉 Example: Intel Pentium series, IBM System/360.

1(b)(ii) Find out the quotient and reminder by performing two's complement division operation where divisor = -4 and dividend = -7. (4 marks)
আমরা এখন two’s complement signed division ওরকমভাবে করব, কিন্তু sign handle করে।

🧮 Given:

- Dividend (Q) = –7

- Divisor (M) = –4

Step 1: Sign আলাদা করি

Dividend negative → sign bit = 1
Divisor negative → sign bit = 1

✅ Sign of Quotient = Dividend ⊕ Divisor = 1 ⊕ 1 = 0 → Positive
✅ Sign of Remainder = Dividend = Negative

Step 2: Unsigned মানে কাজ করবো

|-7| = 7
|-4| = 4

এখন unsigned divide করি:

7 ÷ 4 = 1 remainder 3

Step 3: Sign আবার যোগ করি

- Quotient positive থাকবে (কারণ – ÷ – = +)

- Remainder dividend-এর মতো negative হবে

Step 4: Binary তে দেখাই (4-bit example)

| Value | Decimal | 4-bit Two’s Complement |
| --- | --- | --- |
| Quotient (+1) | 1 | 0001 |
| Remainder (–3) | -3 | invert(0011)=1100+1=1101 |

✅ Final Answer:

- Quotient = +1 → 0001

- Remainder = –3 → 1101

🧠 Easy Summary:

Two’s complement division-এ প্রথমে সবকিছু positive করে divide করা হয়,
শেষে quotient এর sign বের হয় ⊕ দিয়ে,
আর remainder এর sign হয় dividend এর মতো।

Final (Exam-Style Answer):

Quotient = +1 (0001₂)
Remainder = –3 (1101₂)

1(c)(i) (-A + B) in 6-bit Two’s Complement (2 marks)

ধরি,
A = 12 → 001100
B = 20 → 010100

(–A + B) = ?

Step 1: –A = invert(001100) + 1 = 110011 + 1 = 110100
Step 2: Add B

110100

+010100

--------

1001000

6 bit রাখলে → 001000

✅ Answer: (–A + B) = 001000 → 08 (Hex)

👉 সহজভাবে: –12 + 20 = +8 → তাই Hex এ 08।

1(c)(ii) Enlist the categories of computer's functions. What is the difference between arithmetic shift and logical shift? (1+2 marks)

Computer functions:

- Data processing (ডেটা প্রক্রিয়াজাত করা)

- Data storage (ডেটা সংরক্ষণ করা)

- Data movement (I/O কাজ)

- Control operations (সমস্ত process নিয়ন্ত্রণ করা)

Arithmetic shift vs Logical shift:

| Type | Meaning | Example |
| --- | --- | --- |
| Arithmetic Shift | Sign bit change হয় না (used for signed numbers) | 1010 → 1100 |
| Logical Shift | Sign bit ignore হয়, নতুন জায়গায় 0 বসে | 1010 → 0100 |

👉 Easily: Arithmetic shift sign রাখে, Logical shift শুধু bit move করে 0 বসায়।

অবশ্যই 😊 নিচে প্রশ্নসহ পুরো উত্তরগুলো Bangla + English mix করে একদম সহজভাবে দেওয়া হলো —

2(a) Technological features for switching computer generation (4 marks)

Computer generations mainly changed due to technology improvement in switching devices.

| Generation | Technology Used | Features |
| --- | --- | --- |
| 1st (1940–1956) | Vacuum Tubes | বড় size, বেশি heat, slow, huge electricity use |
| 2nd (1956–1963) | Transistors | ছোট, faster, less heat, reliable |
| 3rd (1964–1971) | Integrated Circuits (ICs) | High speed, low cost, more compact |
| 4th (1971–1985) | Microprocessors | All CPU functions on single chip |
| 5th (1985–Present) | VLSI / AI-based processors | Parallel processing, machine learning, AI, quantum computing |

👉 Key technological development (recent consecutive generations):

- 4th → 5th: From microprocessor to AI & VLSI, faster CPUs like Intel i7, multicore.

- 5th → 6th (Current trend): Quantum computing & AI chips (like Tensor cores, Neural Engines) evolving.

2(b)(i) 2’s Complement vs Sign Magnitude (2 marks)

| Point | 2’s Complement | Sign Magnitude |
| --- | --- | --- |
| Representation | Negative = invert bits + 1 | MSB = sign bit (0=+,1=–) |
| Arithmetic | Simple to add/subtract | Complex (need sign check) |
| Zero Representation | Only one zero | Two zeros (+0, –0) |
| Example (4-bit) | +5=0101, –5=1011 | +5=0101, –5=1101 |

→ 2’s complement is mostly used in computers because arithmetic is faster and simpler.

2(b)(ii) Variable Length Instruction (3 marks)

Definition:
সব instruction একই size না হয়ে, কিছু ছোট কিছু বড় হয় — এগুলোকেই variable length instruction বলে।

Example:

MOV R1, R2     → 2 bytes

MOV AX, [1234] → 3 bytes

Small instruction = faster fetching
Large instruction = handles more data/addressing

Issues in Instruction Set Design:

- Complex decoding: CPU কে আলাদা size চিনতে হয়।

- Memory alignment problem: Unequal instruction boundaries.

- Hardware complexity: Variable-length instruction handle করতে harder logic লাগে।

2(c) Condition Codes (2 marks)

Definition:
Condition codes are special flags (Zero, Carry, Negative, Overflow) that show the result status of operations.

Advantages:
✅ Speed up decisions (no need to recalculate conditions)
✅ Useful for branching (e.g., “if zero → jump”)

Disadvantages:
❌ Adds complexity in CPU design
❌ May cause confusion during parallel execution (flags shared)

“Each operation needs operand” – Justify & Operand types (3 marks)

Justification:
Every instruction needs data to work on — this data is called the operand.
Without operand, CPU can’t perform operation like ADD, SUB, LOAD.

Example:
ADD A, B → here A & B are operands.

Categories of Operands:

- Immediate Operand: value directly in instruction
→ e.g., MOV A, 5

- Register Operand: comes from CPU register
→ e.g., ADD R1, R2

- Memory Operand: comes from memory location
→ e.g., LOAD A, [2000]

- I/O Operand: comes from input-output device

✅ Final Short Summary (for exam):

- (a) Generations changed due to new switching devices (Vacuum → Transistor → IC → Microprocessor → AI).

- (b) 2’s complement simpler than sign magnitude; Variable-length instructions = mixed size → complex decoding.

- (c) Condition codes = status flags; operands = data needed for operations.

3(a) Distinguish among Direct, Associative, and Set-Associative Mapping (2 marks)

| Mapping Type | Explanation (Easy) | Example / Feature |
| --- | --- | --- |
| 1️⃣ Direct Mapping | প্রতিটি main memory block কেবলমাত্র একটা নির্দিষ্ট cache line-এ যেতে পারে। | Simple & fast but conflict বেশি হয়। Formula: Cache line = (Block No) mod (No. of lines) |
| 2️⃣ Associative Mapping (Fully Associative) | কোনো memory block cache-এর যেকোনো line-এ store হতে পারে। | Very flexible, conflict কম, but hardware costly & slow searching. |
| 3️⃣ Set-Associative Mapping | Cache কে sets এ ভাগ করা হয়; প্রতিটি memory block যায় নির্দিষ্ট set-এ, কিন্তু সেই set-এর মধ্যে যেকোনো line এ বসতে পারে। | Balance between direct & associative. Example: 2-way, 4-way set associative. |

🧠 Easy summary:

- Direct → One fixed place only.

- Associative → Anywhere in cache.

- Set-Associative → In one set, but flexible inside that set.

✅ Short answer (for 2 marks):

Direct mapping fixes one cache line per block, associative mapping allows any line, and set-associative allows placement within a specific set — giving a balance of speed and flexibility.

3(b)
i. For hexadecimal main memory addresses F000EE, FABCEF, show the following information in hexadecimal format. (Where cache size is 64 Kbytes.)
a. Tag, Line, and Word values for a direct mapped cache.
b. Tag, Set and Word values for two-way set associative cache. (5 marks)

Solve:

Given:

- Main memory addresses: F000EE, FABCEF

- Cache size: 64 KBytes = 64×1024=65536bytes

- Direct mapped cache & 2-way set associative cache

ধরা যাক block size = 1 word = 1 byte (যদি অন্য বলা না থাকে)

(i) Direct Mapped Cache

Step 1: Determine number of cache lines

- Cache size = 64 KB = 65536 bytes

- Block size = 1 byte → Number of lines = 65536 / 1 = 65536 lines

Step 2: Address division

- Main memory address: 24-bit (F000EE, FABCEF → 6 hex digits → 24 bits)

- For direct mapped cache:

Address=[Tag∣Line∣Word]

- Line = log2(#lines) = log2(65536) = 16 bits

- Word = log2(block size) = log2(1) = 0 bits

- Tag = remaining bits = 24 – 16 – 0 = 8 bits

✅ So:

- Tag = upper 8 bits

- Line = next 16 bits

- Word = 0 bits

Step 3: Address F000EE

- Binary: F0 00 EE → Hex

- Tag = F0

- Line = 00EE

- Word = — (0 bits)

Step 4: Address FABCEF

- Hex: FA BC EF

- Tag = FA

- Line = BCEF

- Word = —

(ii) Two-Way Set Associative Cache

Step 1: Determine number of sets

- Two-way → 2 lines per set

- Number of lines = 65536 → Number of sets = 65536 / 2 = 32768 sets

- Set field = log2(#sets) = log2(32768) = 15 bits

- Word = log2(block size) = 0 bits

- Tag = remaining = 24 – 15 – 0 = 9 bits

✅ So:

- Tag = upper 9 bits

- Set = next 15 bits

- Word = 0 bits

তাই, Address = [Tag: 9 bits | Set: 15 bits | Word: 0 bits]

Step 2: Convert F000EE to binary

F000EE Hex → binary 24-bit:

F0 = 1111 0000

00 = 0000 0000

EE = 1110 1110

মিলিয়ে লেখা যাক 24-bit:

1111 0000 0000 0000 1110 1110

- Tag: প্রথম 9 bits = 111100000 → Hex?

  - Binary 111100000 → Decimal 480 → Hex = 1E0

- Set: next 15 bits = 000000011101110 → Binary → Decimal 238 → Hex = 00EE

- Word: 0 bits → —

✅ সুতরাং F000EE → Tag = 1E0, Set = 00EE, Word = —

Step 3: Convert FABCEF to binary

FABCEF Hex → binary:

FA = 1111 1010

BC = 1011 1100

EF = 1110 1111

মিলিয়ে লেখা যাক 24-bit:

1111 1010 1011 1100 1110 1111

- Tag: প্রথম 9 bits = 111110101 → Decimal 501 → Hex = 1F5

- Set: next 15 bits = 011110011101111 → Decimal 15583 → Hex = 3CDF

- Word: 0 bits → —

✅ সুতরাং FABCEF → Tag = 1F5, Set = 3CDF, Word = —

Final Answer (Correct)

| Address | Tag | Set | Word |
| --- | --- | --- | --- |
| F000EE | 1E0 | 00EE | — |
| FABCEF | 1F5 | 3CDF | — |

b-ii) Cache Replacement Algorithms

  - যখন cache এর set পূর্ণ হয়ে যায়, তখন কোন block কে replace করবে সেটা ঠিক করার জন্য কিছু algorithm থাকে। প্রধানগুলো হলো:

  - 1. FIFO (First In First Out)

  - English: Replace the oldest block in the cache first.

  - Bangla: যেটা সবচেয়ে আগে cache এ এসেছে, সেটা প্রথমে replace করা হয়।

  - Note: সহজ, কিন্তু মাঝে মাঝে frequently used block ও replace হয়ে যেতে পারে।

  - Example:
Cache: [A, B, C]
New block D আসলে, oldest A replace হবে।

  - 2. LRU (Least Recently Used)

  - English: Replace the block that has not been used for the longest time.

  - Bangla: যেটা সবচেয়ে দীর্ঘ সময় ধরে use হয়নি, সেটা replace করা হয়।

  - Note: FIFO থেকে better কারণ এটি temporal locality consider করে।

  - Example:
Cache: [A, B, C]
Recent use: A (recent), B, C (long time ago)
New block D আসলে, C replace হবে।

  - 3. LFU (Least Frequently Used)

  - English: Replace the block used least number of times.

  - Bangla: যেটা সবচেয়ে কম বার use হয়েছে, সেটা replace করা হয়।

  - Note: ভাল যদি কিছু block অনেকবার use হয়।

  - Example:
Cache: [A(5 times), B(2 times), C(7 times)]
New block D আসলে, B replace হবে।

  - 4. Random Replacement

  - English: Replace a random block.

  - Bangla: যেকোনো random block replace করা হয়।

  - Note: সহজ, কিন্তু সবসময় optimal নয়।

  - Example:
Cache: [A, B, C]
New block D আসলে, randomly A বা B বা C replace হবে।

  - 💡 সংক্ষেপে:

  - FIFO: সহজ, পুরানো block remove।

  - LRU: বেশি recently used block রাখে।

  - LFU: বেশি frequently used block রাখে।

  - Random: randomly remove।

3(c)(i) Why is SRAM so called? Distinguish between SRAM and DRAM

Why SRAM is called SRAM?

- English: SRAM stands for Static Random Access Memory.

- Bangla: SRAM নামকরণ হয়েছে Static Random Access Memory থেকে।

- Reason: SRAM data কে ধারাবাহিক power থাকা পর্যন্ত static ভাবে রাখে, অর্থাৎ refresh করার দরকার হয় না।

Difference between SRAM and DRAM

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Full Form | Static RAM | Dynamic RAM |
| Data Storage | Stores data statically (without refresh) | Stores data dynamically (needs refresh) |
| Speed | Faster | Slower |
| Cost | Expensive | Cheaper |
| Structure | Uses 6 transistors per bit | Uses 1 transistor + 1 capacitor per bit |
| Power Consumption | Less frequent switching | More due to refresh cycles |
| Usage | Cache memory (CPU) | Main memory (RAM) |

Bangla Summary:

- SRAM: বেশি দ্রুত, বেশি ব্যয়বহুল, refresh লাগে না।

- DRAM: ধীর, সস্তা, refresh লাগে।

3(c)(ii) Explain the SRAM structure

- English: SRAM is made of 6 transistors per bit forming a flip-flop. It stores 1 bit of data in a stable state.

- Bangla: SRAM এ প্রতি bit 6টি transistor দিয়ে তৈরি হয়। এগুলো flip-flop হিসেবে কাজ করে এবং একটিমাত্র bit stable ভাবে রাখে।

- Access: Read/write সহজ এবং fast, কারণ refresh দরকার নেই।

Simple Diagram idea:

Q ----|>---+

Q' ---|>---+

(cross-coupled inverters form the flip-flop)

- Q = 1 বা 0 state ধরে রাখে।

- Access transistors connect bit line during read/write।

3(d) Distinguish between EPROM and EEPROM

| Feature | EPROM | EEPROM |
| --- | --- | --- |
| Full Form | Erasable Programmable ROM | Electrically Erasable Programmable ROM |
| Erase Method | UV light used to erase | Electric signal used to erase |
| Rewrite | Slower, needs UV erase | Faster, can erase/write electrically |
| Usage | Firmware that rarely changes | Firmware that may need frequent updates |
| Cost | Cheaper | More expensive |
| Convenience | Less convenient | More convenient |

Bangla Summary:

- EPROM: UV light দিয়ে erase, ধীরে update।

- EEPROM: Electric signal দিয়ে erase, সহজে update।

💡 Tip: SRAM, DRAM, EPROM, EEPROM সবই memory এর প্রকার। মূল পার্থক্য হলো speed, erase method, refresh requirement, structure।

4(a) What is RAID? Explain error detection and error correction mechanisms in semiconductor memory

RAID

- English: RAID = Redundant Array of Independent/Inexpensive Disks.

- Bangla: RAID হলো একাধিক হার্ড ডিস্ককে একসাথে যুক্ত করে ডেটা সংরক্ষণের একটি পদ্ধতি, যাতে data loss কমানো যায় এবং performance বাড়ে।

- Purpose:

  - Data redundancy (backup)

  - Faster read/write

Error Detection and Error Correction in Semiconductor Memory

- Error Detection

  - Detects if memory data has changed accidentally.

  - Methods:

    - Parity bit (extra bit 0 বা 1 add করা হয়)

    - Checksum (sum of data bits check)

- Error Correction

  - Automatically correct the wrong data.

  - Methods:

    - Hamming code (can correct 1-bit error)

    - ECC (Error Correcting Code) memory

Bangla Summary:

- Error detection: ভুল ধরার জন্য।

- Error correction: ভুল ঠিক করার জন্য।

4(b) Technological features of storage devices

| Device | Feature (Bangla + English) |
| --- | --- |
| Tape | Sequential access, cheap, large storage, slow read/write |
| CD | Optical storage, 700 MB, read mostly, portable |
| DVD | Optical storage, 4.7–8.5 GB, video/data, read/write possible |
| HDD | Magnetic storage, fast access, mechanical spinning platters |
| SSD | Solid-state, fast, no moving parts, more expensive |
| Flash | Non-volatile, small, portable, used in USB, memory cards |

4(c) Why I/O module is used & its functions

Why?

- English: To interface CPU with input/output devices.

- Bangla: CPU সরাসরি I/O device এর সাথে communicate করতে পারে না, তাই I/O module ব্যবহার করা হয়।

Functions of I/O module

- Data transfer between CPU and I/O devices

- Device control (start/stop operations)

- Buffering (temporary storage)

- Error detection in I/O

Bangla Summary:

- Data পাঠানো/গ্রহণ করা, device control, temporary storage, error check।

4(d) Peripheral devices & classifications

Peripheral devices

- Devices outside the CPU & memory, like keyboard, mouse, printer, scanner.

Classification

- Input devices: Keyboard, mouse, scanner

- Output devices: Monitor, printer, speaker

- Input/Output devices: Touchscreen, network card

Program-driven vs Interrupt-driven I/O

| Feature | Program-driven | Interrupt-driven |
| --- | --- | --- |
| CPU waits for device | Yes | No |
| Efficiency | Low | High |
| Example | Polling loop | Keyboard interrupt |

5(a)(i) Describe pipelining & branch handling

Pipelining

- English: CPU executes multiple instructions simultaneously in stages.

- Bangla: একাধিক instruction কে ধাপে ধাপে একসাথে process করা হয়, যেমন assembly line।

Branch Handling

- Problem: Branch instruction uncertainty can stall pipeline.

- Solution:

  - Branch prediction

  - Delayed branching

  - Flush pipeline if mispredicted

5(a)(ii) Pipeline processor speed

- Statement: "k-stage pipeline processor is k times faster than non-pipeline."

- Reason:

  - Non-pipeline: executes 1 instruction at a time.

  - Pipeline: executes k instructions in k stages simultaneously → increases throughput ≈ k times.

5(b)(i) Interrupts purpose & classification

Why interrupts?

- English: To let CPU handle important events immediately.

- Bangla: CPU কে তাত্ক্ষণিক কাজ করার সুযোগ দেয়, যেমন key press বা device ready।

Classification

- Hardware interrupt: From devices (keyboard, mouse)

- Software interrupt: From programs (system call)

- Maskable vs Non-maskable: CPU can ignore maskable, must respond to non-maskable

5(b)(ii) Handling multiple interrupts

- Priority scheme: Assign priority to each interrupt

- Interrupt nesting: Higher priority interrupt can interrupt a lower priority one

- Interrupt vector table: CPU looks up which routine to run

5(c)(i) Addressing mechanisms

- Direct addressing

- Indirect addressing

- Immediate addressing

- Register addressing

- Indexed addressing

- Base + displacement

5(c)(ii) Effective address EA = (A) + (R)

- Formula: EA = Base + Displacement

- A = Base value (memory start address)

- R = Register holding offset/displacement

Example:

- Base (A) = 1000

- Register R = 20

- EA = 1000 + 20 = 1020

Bangla:

- Base address + register offset = final memory address (effective address)

💡 Tip for exams:

- সব memory, I/O, pipeline, interrupts-এর tables বা diagrams মনে রাখা সহজ।

- Addressing এর জন্য ছোট example লিখে দেখাও।

ঠিক আছে, আমি তোমাকে সহজ বাংলায় + ইংরেজি মিলিয়ে step by step বুঝিয়ে দিচ্ছি।

6(a)(i) Number of Page Table Entries (PTE)

Formula:
[
\text{Number of PTE} = \frac{\text{Virtual Address Space}}{\text{Page Size}} = \frac{2^n}{2^p} = 2^{n-p}
]

| Virtual Address n | Page Size P | PTE = 2^(n-p) |
| --- | --- | --- |
| 32-bit | 4 KB = 2^12 | 2^(32-12) = 2^20 = 1,048,576 |
| 64-bit | 8 KB = 2^13 | 2^(64-13) = 2^51 ≈ 2.25 × 10^15 |

Bangla summary:

- PTE = কতগুলো page mapping লাগবে, সেটা virtual address এবং page size থেকে হিসাব করা হয়।

6(a)(ii) Page Fault Handling in Virtual Memory

Steps:

- CPU accesses a page → page not in main memory → page fault occurs.

- OS saves the current state of the process (trap to OS).

- OS finds a free frame in main memory.

- If no free frame → replace a page using page replacement algorithm (FIFO, LRU, etc.).

- Load the required page from disk to main memory.

- Update page table with new frame number.

- Resume process from where it was interrupted.

Bangla:

- Page fault হলে OS disk থেকে missing page memory-তে load করে, page table update করে process resume করে।

6(b) Distinguish between CISC and RISC

| Feature | CISC (Complex Instruction Set Computer) | RISC (Reduced Instruction Set Computer) |
| --- | --- | --- |
| Instruction length | Variable | Fixed |
| Instruction set | Complex, many instructions | Simple, few instructions |
| Execution time | Multiple cycles | Usually 1 cycle per instruction |
| Hardware | Complex | Simple |
| Example | Intel x86 | ARM, MIPS |

Bangla Summary:

- CISC: বড়, জটিল instructions, slow, heavy hardware

- RISC: ছোট, simple instructions, fast, efficient

6(c) Micro-operations for IAS architecture

Instruction: ADD AX, 3

Steps (simplified IAS style):

- Fetch:

  - MAR ← PC (Memory Address Register gets PC)

  - MDR ← M[MAR] (Memory Data Register gets instruction)

  - IR ← MDR (Instruction Register stores instruction)

  - PC ← PC + 1

- Decode:

  - Decode IR to identify opcode = ADD, operand = AX, 3

- Execute:

  - AC ← AC + 3 (Accumulator adds 3)

Bangla Summary:

- Fetch → Decode → Execute → Update AC

6(d) CPU Performance Calculation

Given:

| Instruction Type | % occurrence | Cycles per Instruction |
| --- | --- | --- |
| ALU | 40% | 2 |
| Load & Store | 17% | 3 |
| Branch | 40% | 5 |
| Others | 3% | 6 |

- Total instructions = 350

- Clock rate = 1.2 GHz = 1/1.2e9 sec per cycle

💡 Summary Table:

| Parameter | Value |
| --- | --- |
| CPI | 3.49 |
| CPU Time | 1.018 µs |
| MIPS | 344 |

Bangla summary:

- CPI = গড় cycles per instruction

- CPU Time = total execution time

- MIPS = million instructions per second