Previous solve – 18-19

1(a) RAID Working Mechanism

RAID (Redundant Array of Independent/Inexpensive Disks)

- English: RAID combines multiple disks to improve performance and/or fault tolerance.

- Bangla: একাধিক হার্ড ডিস্ক একসাথে যুক্ত করে ডেটা রাখা হয় যাতে দ্রুত পড়া/লেখা সম্ভব হয় এবং ডেটা হারানো কম হয়।

Working Mechanism:

- Data Striping: Data কে ছোট ছোট block এ ভাগ করে multiple disks এ রাখে → speed বৃদ্ধি

- Mirroring: একই data একাধিক disk এ রাখে → fault tolerance বৃদ্ধি

- Parity: Extra parity bit/blocks ব্যবহার করে error detection & recovery

Bangla Summary:

- Striping → দ্রুত

- Mirroring → নিরাপদ

- Parity → ভুল ধরার ও ঠিক করার জন্য

1(b) Bus Performance & Types

Why performance suffers with many devices:

- English: If many devices share a bus, each device waits for its turn → slower communication.

- Bangla: অনেক device একসাথে bus use করলে, প্রতিটা device পরপর অপেক্ষা করতে হয় → system ধীর হয়ে যায়।

Types of Bus

| Bus Type | Function | Bangla Explanation |
| --- | --- | --- |
| Local Bus | Connects CPU directly with high-speed devices (like RAM, graphics) | CPU কে fast device সাথে যুক্ত করে, speed বেশি |
| System Bus | Connects CPU, memory, and I/O devices | CPU, memory, I/O device এর মূল connection |
| Expansion Bus | Connects slower devices via slots (PCI, ISA, USB) | Extra devices যেমন printer, scanner connect করতে use হয়, speed কম |

1(c) Characteristics of RISC & CISC

| Feature | RISC | CISC |
| --- | --- | --- |
| Instruction Set | Simple, few instructions | Complex, many instructions |
| Execution | Usually 1 cycle per instruction | Multiple cycles per instruction |
| Instruction Length | Fixed | Variable |
| Hardware | Simple | Complex |
| Example | ARM, MIPS | Intel x86 |

Bangla Summary:

- RISC: ছোট, simple, fast, কম hardware

- CISC: বড়, জটিল, slow, বেশি hardware

1(d) Set Associative Mapping & Replacement Algorithms

Set Associative Mapping:

- English: Memory block → specific set in cache → any block in that set.

- Bangla: Cache কে multiple sets এ ভাগ করা হয়, main memory block ঠিক set এ যায়, কিন্তু set এর মধ্যে যেকোনো block রাখা যায়।

Example:

- Cache = 8 blocks, 2-way set associative → 4 sets × 2 blocks/set

- Memory block → maps to one set, choose 1 block from 2 to replace

Replacement Algorithms:

- FIFO (First In First Out): পুরানো block replace

- LRU (Least Recently Used): দীর্ঘ সময় ধরে ব্যবহার না হওয়া block replace

- LFU (Least Frequently Used): সবচেয়ে কম ব্যবহার হওয়া block replace

- Random Replacement: Randomly block replace

Bangla Summary:

- FIFO → সহজ, কখনো inefficient

- LRU → recent use consider করে

- LFU → frequent use consider করে

- Random → random replace, সবসময় best নয়

💡 Quick Tip:

- RAID = speed + safety

- Bus = connection + speed

- RISC vs CISC = simple vs complex

- Cache mapping & replacement = set → select block → replace algorithm

2(a) OS working mechanism (User ↔ Hardware)

- English: Operating System (OS) acts as an interface between user and hardware.

- Bangla: OS হলো মধ্যবর্তী layer, যা ব্যবহারকারীর command কে hardware বুঝতে সাহায্য করে এবং hardware এর result ব্যবহারকারীর কাছে পৌঁছে দেয়।

Working Mechanism:

- User gives command (keyboard, mouse, GUI)

- OS interprets command → converts to hardware instructions

- Hardware executes the instructions (CPU, memory, I/O devices)

- Result sent back to user via OS

Bangla summary:

- User → OS → Hardware → OS → User

2(b) Two’s complement division: divisor = -3, dividend = -7

Step 1: Represent numbers in 4-bit (example)

- 3 = 0011 → -3 (Two’s complement) = 1101

- 7 = 0111 → -7 (Two’s complement) = 1001

Step 2: Division rule (Two’s complement)

- Sign of quotient = sign of dividend XOR sign of divisor

- Sign of remainder = same as dividend

Step 3: Calculate absolute division

- |7| ÷ |3| → Quotient = 2, Remainder = 1

Step 4: Apply signs

- Dividend = -7, Divisor = -3 → quotient = +2, remainder = -1

✅ Answer:

- Quotient = 2

- Remainder = -1

2(c) Random Access Memory (RAM) & Number Representations

RAM definition

- English: RAM = Memory which can be read and written in any order.

- Bangla: RAM হলো এমন memory যা থেকে যেকোনো address থেকে data পড়া বা লেখা যায়, sequential এর প্রয়োজন নেই।

Two’s complement vs Sign magnitude

| Feature | Two’s Complement | Sign Magnitude |
| --- | --- | --- |
| Representation of negative | Invert + 1 | MSB = sign bit (1 = negative, 0 = positive) |
| Arithmetic | Easy (addition/subtraction same as positive) | Hard (need separate logic for subtraction) |
| Zero | Single zero | Two zeros (+0, -0) |
| Example: -5 (4-bit) | 1011 | 1001 |

Bangla summary:

- Two’s complement → calculation সহজ, এক zero

- Sign magnitude → calculation কঠিন, দুই zero

2(d) Direct Mapped Cache Problem

Given:

- Main memory = 2¹⁶ bytes

- Block size = 8 bytes → 3 bits for byte offset (2³ = 8)

- Cache lines = 32 → 5 bits for line number (2⁵ = 32)

- Tag bits = remaining → 16 - 5 - 3 = 8 bits

2(d)(i) Memory address division

- Tag = 8 bits

- Line number = 5 bits

- Byte number = 3 bits

2(d)(ii) Cache line calculation

Memory address format:

[Tag (8)] [Line (5)] [Byte (3)]

- Address = 0001 0001 0001 1011

- Tag = 00010001

- Line = 00011 → Decimal = 3

- Byte = 011 → Decimal = 3

- Address = 1100 0011 0011 0100

- Tag = 11000011

- Line = 00110 → Decimal = 6

- Byte = 100 → Decimal = 4

✅ Answer:

- First address → Line 3

- Second address → Line 6

2(d)(iii) Total cache bytes

- Cache lines = 32

- Block size = 8 bytes
[
\text{Total cache bytes} = 32 × 8 = 256 \text{ bytes}
]

💡 Summary Table:

| Parameter | Value |
| --- | --- |
| Byte offset | 3 bits |
| Line number | 5 bits |
| Tag | 8 bits |
| Line for 0001 0001 0001 1011 | 3 |
| Line for 1100 0011 0011 0100 | 6 |
| Total cache memory | 256 bytes |

ঠিক আছে! আমি তোমাকে সহজ বাংলায় + ইংরেজি মিশিয়ে step by step বোঝাচ্ছি।

3(a) Program-driven vs Interrupt-driven I/O

| Feature | Program-driven I/O | Interrupt-driven I/O |
| --- | --- | --- |
| CPU waiting | CPU continuously checks device (polling) | CPU is interrupted when device is ready |
| Efficiency | Low, wastes CPU cycles | High, CPU can do other tasks |
| Example | Loop waiting for keyboard input | Keyboard generates interrupt when key pressed |
| Bangla Summary | CPU সবসময় device কে check করে → slow | Device ready হলে interrupt দেয় → efficient |

3(b)(i) Micro-operations to add two memory operands

Instruction: ADD M1, M2 (result stored in AC/Accumulator)

Steps:

- Fetch instruction:

  - MAR ← PC

  - MDR ← M[MAR]

  - IR ← MDR

  - PC ← PC + 1

- Fetch operands:

  - MAR ← Address of M1

  - MDR ← M[MAR]

  - AC ← MDR (AC stores first operand)

  - MAR ← Address of M2

  - MDR ← M[MAR]

- Execute addition:

  - AC ← AC + MDR

- Store result (optional):

  - M[Address of result] ← AC

Bangla Summary:

- Fetch → Operand load → Execute → Store

3(b)(ii) Two’s complement addition (6-bit)

Given: -1A + 2B (Hex)

Step 1: Convert Hex to binary (6-bit)

- 1A Hex → 011010 → -1A → Two’s complement → invert +1 → 100110

- 2B Hex → 101011

Step 2: Add binary numbers

100110  (-1A)

+ 101011  (2B)

-----------

010001  (Ignore carry beyond 6 bits)

Step 3: Convert result back to Hex

- 010001 = 11 Hex

✅ Answer: 11 Hex

3(c) I/O Module & External Device Classification

Why I/O module is used?

- English: To interface CPU with external devices.

- Bangla: CPU সরাসরি device handle করতে পারে না → I/O module CPU ↔ device manage করে।

Functions:

- Data transfer between CPU and device

- Device control

- Buffering

- Error detection

External Device Classification:

| Type | Examples | Bangla |
| --- | --- | --- |
| Input | Keyboard, Mouse, Scanner | Data input |
| Output | Monitor, Printer, Speaker | Data output |
| Input/Output | Network card, Touchscreen | Data both ways |

3(d) Short Notes

i. Dual-core

- English: Processor with two cores on a single chip → can execute two tasks simultaneously.

- Bangla: এক processor chip এ দুইটি core → একই সময়ে দুইটি কাজ করতে পারে → speed বৃদ্ধি।

ii. Pentium 4

- English: Intel CPU released in early 2000s, high clock speed, single-core, used NetBurst architecture.

- Bangla: পুরোনো Intel processor, দ্রুত clock, single core, performance-oriented।

iii. Core i7

- English: Modern Intel CPU, multi-core, hyper-threading, high performance for desktop & laptop.

- Bangla: Intel এর নতুন generation, multiple cores + threads, high-speed processing, gaming & professional use।

💡 Quick Tip:

- Program-driven = CPU polling → slow

- Interrupt-driven = CPU interrupted → fast

- I/O module → CPU ↔ devices interface

- Dual-core/Core i7 → modern multi-tasking, high speed

- Pentium 4 → older Intel CPU

4(a) Computer Organization vs Architecture & Main Structural Components

1. Computer Architecture vs Organization

| Feature | Computer Architecture | Computer Organization |
| --- | --- | --- |
| Definition | Logical design of a computer, what a computer does | Physical implementation, how it is built |
| Example | Instruction set, addressing modes | ALU design, memory circuits, bus structure |
| Bangla summary | Architecture = কম্পিউটার কি করে | Organization = কম্পিউটার কিভাবে কাজ করে |

2. Main Structural Components of a Computer

- CPU (Central Processing Unit) – Performs calculations and controls operations.

- Memory – Stores data and instructions.

- Input Devices – Provide data to the computer (keyboard, mouse).

- Output Devices – Display results (monitor, printer).

- Control Unit (CU) – Directs data flow between CPU, memory, and I/O.

- ALU (Arithmetic Logic Unit) – Performs arithmetic & logic operations.

- Registers – Small, fast storage inside CPU for temporary data.

Bangla summary:

- CPU + Memory + I/O + CU + ALU + Registers → computer এর main parts।

4(b) Moore’s Law & IAS Memory Read/Write

1. Moore’s Law

- English: The number of transistors on a chip doubles approximately every 18–24 months.

- Bangla: একটি chip এ transistor সংখ্যা প্রায় প্রতি ১.৫–২ বছরে দ্বিগুণ হয়, → speed এবং performance বৃদ্ধি পায়।

2. IAS Memory Read/Write Process

Components:

- MAR (Memory Address Register) – Holds address to access memory

- MBR (Memory Buffer Register) – Holds data read from or to be written to memory

- Address Bus – Carries address from CPU to memory

- Data Bus – Carries data between CPU and memory

- Control Bus – Sends read/write signals

Memory Read:

- MAR ← Address of memory

- Control bus ← Read signal

- Memory sends data → Data bus → MBR

- CPU uses data from MBR

Memory Write:

- MAR ← Address of memory

- MBR ← Data to write

- Control bus ← Write signal

- Data bus sends data → memory

Bangla summary:

- Read → MAR address, Read signal, MBR gets data

- Write → MAR address, MBR data, Write signal → memory

4(c) Key Properties of Semiconductor Memory & DRAM vs SRAM

1. Key Properties

- Volatility: Loses data when power off (for RAM)

- Access time: Speed to read/write data

- Density: Number of bits per chip

- Cost: Price per bit

- Power consumption

2. DRAM vs SRAM

| Feature | DRAM | SRAM |
| --- | --- | --- |
| Storage | 1 transistor + 1 capacitor per bit | 6 transistors per bit |
| Speed | Slower | Faster |
| Cost | Cheaper | Expensive |
| Refresh | Needs refresh | No refresh |
| Use | Main memory | Cache memory |

Bangla summary:

- DRAM → ধীরে, সস্তা, refresh লাগে

- SRAM → দ্রুত, ব্যয়বহুল, refresh লাগে না

4(d) Memory Cell Definition & Structure

1. Memory Cell

- English: Smallest unit in memory that stores 1 bit of data.

- Bangla: Memory cell হলো smallest unit যা 1 bit data রাখে।

2. Typical Memory Cell Structure

- SRAM cell: 6 transistors forming a flip-flop

- DRAM cell: 1 transistor + 1 capacitor

- Function: Stores 0 or 1 and allows read/write via word line and bit line

Simple diagram idea (SRAM flip-flop):

Q ----|>---+

Q' ---|>---+

(cross-coupled inverters store 1 bit)

Bangla summary:

- SRAM → fast, transistor-based

- DRAM → capacitor + transistor, needs refresh

💡 Quick Tip:

- Architecture = কি করে

- Organization = কিভাবে করা হয়

- Moore’s Law → chip speed + transistor growth

- IAS read/write → MAR, MBR, buses, control signal

- Memory → DRAM (main), SRAM (cache), cell = 1 bit storage

5(a) Chip Logic & 16M × 4 DRAM Structure

1. What is Chip Logic?

- English: Chip logic refers to the internal circuitry and logic gates on a memory or processor chip that perform operations like storing, reading, and writing data.

- Bangla: Chip logic হলো একটি chip-এর ভিতরের circuitry, যা data read/write, address decoding, এবং control signals পরিচালনা করে।

2. 16 Megabit DRAM (4M × 4) Structure

- 16 Megabit DRAM → 16 × 2²⁰ bits = 16,777,216 bits

- Organization: 4M × 4

  - 4M = 4 × 2²⁰ = 4,194,304 words

  - Each word = 4 bits → 4 × 4,194,304 = 16,777,216 bits

Structure:

- Memory divided into rows and columns (2D array)

- Row decoder + Column decoder selects specific word

- 4-bit data bus transfers data to/from CPU

- Each cell = 1 transistor + 1 capacitor (DRAM)

Bangla summary:

- 16Mbit = 4M words × 4 bits

- Row & column decoder selects word

- Data bus = 4 bits wide

5(b) Error-Correcting Code (ECC) & Hamming Code

1. How ECC works

- English: ECC adds redundant bits (parity bits) to memory data to detect and correct errors.

- Bangla: ECC extra bits যোগ করে, যা ব্যবহার করে 1-bit বা 2-bit error detect এবং correct করা যায়।

2. Hamming Code Procedure

- Step 1: Place parity bits at positions 1, 2, 4, 8…

- Step 2: Calculate parity bits such that each parity bit checks a unique combination of data bits

- Step 3: If a single bit error occurs, the combination of parity errors identifies the exact bit in error

- Step 4: Flip the wrong bit → error corrected

Bangla summary:

- Hamming code → detect + correct single-bit error

- Parity bits → error location 알려준다

5(c) Elements of a Machine Instruction & Instruction Cycle Diagram

1. Elements of a Machine Instruction

- Opcode – Operation to perform (ADD, SUB, LOAD)

- Operand(s) – Data or memory/register addresses

- Addressing Mode – How to interpret operand

- Optional Control Bits – Condition codes, immediate flags

2. Instruction Cycle State Diagram

States:

- Fetch – Get instruction from memory → IR

- Decode – Determine opcode & operands

- Execute – Perform operation in ALU / memory

- Store / Write Back – Save result to destination

Diagram (simplified):

[Fetch] → [Decode] → [Execute] → [Store/Write Back] → [Fetch]...

Bangla summary:

- CPU continually repeats Fetch → Decode → Execute → Store

5(d) Nested Procedures & Shifts

1. Nested Procedures

- English: A procedure (function) that calls another procedure or even itself (recursion).

- Bangla: Nested procedure হলো একটি function অন্য function কে call করে বা নিজেকেও call করতে পারে।

- Example:

void A() { B(); }

void B() { ... }

2. Arithmetic vs Logical Shift

| Feature | Arithmetic Shift | Logical Shift |
| --- | --- | --- |
| Shifts bits left/right | Preserves sign bit (for signed numbers) | Fills with 0 (ignores sign) |
| Use | Signed arithmetic | Unsigned numbers, bit manipulation |
| Example | 1010 (−6, 4-bit) >> 1 → 1101 (−3) | 1010 >> 1 → 0101 |

Bangla summary:

- Arithmetic shift → sign preserved

- Logical shift → 0 দিয়ে fill

💡 Quick Exam Tip:

- 16M × 4 DRAM = 4M words × 4 bits → row/column + 4-bit bus

- Hamming code → parity bits detect & correct errors

- Machine instruction → opcode + operand + addressing + optional

- Instruction cycle = Fetch → Decode → Execute → Store

- Nested procedure = function calling another

- Arithmetic shift = signed, logical shift = unsigned

ঠিক আছে! আমি তোমাকে সহজ বাংলা + ইংরেজি মিলিয়ে step by step বুঝাই।

6(a) Register Addressing & x86 Instruction Formats

1. Register Addressing

- English: Register addressing is when the operand of an instruction is located in a CPU register instead of memory.

- Bangla: Register addressing হলো যখন instruction এর data CPU এর internal register থেকে নেওয়া হয়, memory access লাগে না → faster।

Example: ADD AX, BX → both operands in registers.

2. x86 Instruction Formats

- x86 instructions can have variable formats:

- One-byte opcode – simplest instruction (NOP)

- One-byte opcode + Register/Memory operand (ADD AX, [1000H])

- Opcode + ModR/M byte + Displacement/Immediate data

  - ModR/M byte specifies register/memory

  - Immediate data – constant value

- Bangla summary: x86 instruction format = opcode + operand + optional displacement/immediate

6(b) User-Visible Registers & CPU Structure

1. User-Visible Registers

- English: Registers that programmers can directly access via instructions.

- Bangla: CPU register যা programmer instruction দিয়ে read/write করতে পারে।

Examples:

- General purpose: AX, BX, CX, DX (x86)

- Index/Pointer: SI, DI, BP, SP

2. Internal Structure of CPU

Components:

- ALU – arithmetic & logic operations

- Registers – temporary storage (general + special)

- Control Unit (CU) – fetch/decode/execute control

- Instruction Register (IR) – stores current instruction

- Program Counter (PC) – address of next instruction

- Memory Address Register (MAR) – holds memory address

- Memory Buffer Register (MBR) – data to/from memory

Diagram (simplified):

+-----------+

|  Control  |

|   Unit    |

+-----+-----+

|

+--------------+--------------+

|             ALU             |

+--+--+--+--+--+--+--+--+--+--+

|AX|BX|CX|DX|SP|BP|SI|DI|... |

+-----------------------------+

6(c) Control & Status Registers & Data Flow

1. Control Registers

- English: Registers that control CPU operation.

- Bangla: CPU এর behavior নিয়ন্ত্রণ করে।

- Examples:

  - PC (Program Counter) – next instruction address

  - IR (Instruction Register) – current instruction

  - MAR / MBR – memory access

  - Status Register / Flags – zero, carry, sign

2. Status Registers

- Contain flags that show results of operations:

  - Zero flag (Z), Carry flag (C), Sign flag (S), Overflow flag (O)

3. Data Flow of Fetch & Indirect Cycle

Fetch Cycle:

- PC → MAR (address of instruction)

- Memory → MBR → IR

- PC ← PC + 1

Indirect Cycle (if instruction uses memory operand):

- MAR ← effective address

- Memory → MBR (load operand)

Bangla summary:

- Fetch → instruction memory থেকে IR এ

- Indirect → operand memory থেকে MBR এ

6(d) Instruction Pipelining & Six-Stage Pipeline

1. Instruction Pipelining

- English: Technique to overlap execution of multiple instructions in CPU stages to improve throughput.

- Bangla: CPU multiple instruction একসাথে বিভিন্ন stage এ process করে → faster execution।

2. Six-Stage Pipeline

| Stage | Function |
| --- | --- |
| IF | Instruction Fetch |
| ID | Instruction Decode |
| OF | Operand Fetch |
| EX | Execute |
| MA | Memory Access |
| WB | Write Back |

3. Timing Diagram (simplified)

Time →

Instr1: IF ID OF EX MA WB

Instr2:    IF ID OF EX MA WB

Instr3:       IF ID OF EX MA WB

- Each instruction overlaps in different stages

- Throughput increases → execution faster

Bangla summary:

- Six stages → IF, ID, OF, EX, MA, WB

- Each instruction overlapped → pipeline efficiency

💡 Quick Tip Table:

| Topic | Key Points |
| --- | --- |
| Register Addressing | Operand in CPU register → fast |
| x86 Format | Opcode + ModR/M + displacement/immediate |
| User-visible Register | AX, BX, CX, DX, SI, DI |
| Control/Status | PC, IR, MAR, MBR, Flags |
| Fetch Cycle | PC → MAR → MBR → IR → PC+1 |
| Pipeline | 6 stages → IF ID OF EX MA WB → overlapping execution |