COA PREVIOUS QUESTION

SESSION – (19-20)

1.

a) Give the learning outcomes from this course with respect to your current and forthcoming professional perspective. (4 marks)
b)
i. What is computer family? (1 mark)
ii. Find out the quotient and reminder by performing two's complement division operation where divisor = -4 and dividend = -7. (4 marks)
c)
i. Assume numbers are represented in 6-bit two's complemented representation. Show calculation of the following. (-A + B) (Hex) (2 marks)
ii. Enlist the categories of computer's functions. What is the difference between arithmetic shift and logical shift? (1+2 marks)

2.

a) Give the technological features for switching computer generation. Also, mention the key technological development for any 2 consecutive generation in current era. (4 marks)
b)
i. Give the comparative discussion between 2's complement and sign magnitude representation. (2 marks)
ii. Explain variable length instruction with example. Explain the issues regarding an instruction set design. (3 marks)
c) Mention the advantages and disadvantages of condition codes. "Each operation needs operand"-justify this statement. Briefly describe various categories of operand. (3 marks)

- a) Distinguish among direct, associative and set associative mapping of cache. (2 marks)
b)
i. For hexadecimal main memory addresses F000EE, FABCEF, show the following information in hexadecimal format. (Where cache size is 64 Kbytes.)
a. Tag, Line, and Word values for a direct mapped cache.
b. Tag, Set and Word values for two-way set associative cache. (5 marks)
ii. Explain different replacement algorithms with respect to cache memory. (2.5 marks)
c)
i. Why is SRAM so called? Distinguish between SRAM and DRAM. (2 marks)
ii. Explain the SRAM structure. (2 marks)
d) Distinguish between EPROM, EEPROM. (2 marks)

- a) What is RAID? Explain error detection and error correction mechanisms in semiconductor memory. (4 marks)
b) Give the technological features of the storage tape, CD, DVD, HDD, SSD, flash. (2 marks)
c) Why input/output module is used in computer system? Describe the functions of I/O module. (2+2 marks)
d) Briefly explain peripheral devices with its classification. Distinguish between program driven and interrupt driven I/O. (2+2 marks)

- a)
i. Describe pipelining. How can the pipeline be sustained in case of branch dealing? (1+2 marks)
ii. "Pipeline processor with k stages is k times faster than non-pipeline processor." Justify this statement. (2 marks)
b)
i. Why interrupts are used in computer system? Classify interrupts. (2.5 marks)
ii. How does the processor handle multiple interrupts? (2.5 marks)
c)
i. Enlist different addressing mechanisms. (1.5 marks)
ii. Illustrate the effective address EA = (A) + (R), where A = base value, R = register that holds displacement. (2.5 marks)

- a)
i. Determine the number of page table entries that are needed for the following combinations of virtual address size (n) and page size (P). (1+3 marks)

| n | P=2^P | #PTE |
| --- | --- | --- |
| 32 | 4K |  |
| 64 | 8K |  |

- ii. How does the page fault handle in VM system? (3 marks)

b) Distinguish between CISC and RISC. (3 marks)

c) Explain the micro-operations for executing the following instruction with respect to IAS architecture.
ADD AX, 3 (3 marks)

d) The following performance measures were recorded when executing a set of benchmark programs for a machine:

| Instruction category | Percentage of occurrence | No. of cycles per instruction |
| --- | --- | --- |
| ALU | 40% | 2 |
| Load & Store | 17% | 3 |
| Branch | 40% | 5 |
| Others | 3% | 6 |

Assume that the execution of 350 instructions and the clock rate of the CPU is 1.2 GHz. Calculate CPU time, CPI, and MIPS. (1+3 marks)

20-21

- (a) Define Computer Organization and Computer Architecture. What is the difference between them? (2 marks)

(b) What are the key concepts of Von Neumann architecture? Define hardware and software programming. (5 marks)

c)  Define Interrupts. What are the approaches for handling multiple interrupts? Illustrate and explain. (5 marks)

(d) What are the steps of the instruction cycle for the instruction "SUB B, A," which stores the difference of memory locations B and A into location A? (2 marks)

2.

a) Define computer instructions and describe their types. (3 marks)
b) Where Zero-address instructions are applicable and how? Explain. (2 marks)
c) What are logical, arithmetic, and cyclic (rotate) shifts? Apply a 4-bit logical right shift, logical left shift, arithmetic right shift, arithmetic left shift, and left rotate to the bits 10100110. (5 marks)
d) What is a procedure, and why is it necessary? Illustrate and explain the procedure mechanism. (4 marks)

  - (a) What is an I/O module, and why is it necessary? (3 marks)

(b) What are the differences among Programmed I/O, Interrupt-driven I/O, and direct memory access (DMA)? Describe with flowcharts. (5 marks)

(c)How does a DMA module transfer control to the processor after its task? Does it interrupt the processor? (2 marks)

- Define cycle stealing in DMA. (2 marks)

- Give the features of CISC and Core i7 in the perspective of computer architecture and organization. (4 marks)

Here is the extracted text from the image you uploaded:

a) A hypothetical machine has 6 instructions:
1110 = Load AC from memory
0001 = Store AC to memory
0101 = Add to AC from memory
1011 = Sub memory from AC
0100 = Store AC to I/O
1101 = Load AC from I/O

In this case, the 12-bit address identifies a particular I/O device. Show the program execution steps (using 16-bit instruction format where prefix 4 bits for opcode) for the following program:

i. Load AC from device 99
ii. Add contents of memory location 555
iii. Store AC to memory location 666

b) Explain RAID 2. Mention the characteristics of semiconductor memories (4 types). (2+2 marks)
c) Enlist different addressing modes. (2+3 marks)
d) Assume numbers are represented in 6-bit two’s complemented representation. Show calculation of the following:
a) -A + B (Hex)
b) -33 - 17 (Decimal)

5)
a) Consider a machine with a byte addressable main memory of 2^16 bytes and block size of 8 bytes. Assume that a direct mapped cache consisting of 32 lines is used with this machine.
i. How is a 16-bit memory address divided into tag, line number, and byte number?
ii. Into what line would bytes with each of the following addresses be stored?

- 0001 0001 0001 1011

- 1101 0000 0001 1101
iii. Suppose the byte with address 0001 1010 0001 1010 is stored in the cache. What are the addresses of the other bytes stored along with it?
iv. How many total bytes of memory can be stored in the cache? (5 marks)

b) Calculate the multiplication operation using Booth's algorithm for Q = -17 (multiplicand) and M = 16 (multiplier). (4 marks)

c) Define access time and cycle time. (2+3 marks)
d) Distinguish among sequential, direct, and random memory access. (2+3 marks)

Here is the extracted text from the image you uploaded:

6
a)
i. Explain the responsibility of memory management unit in computer systems. (2+3 marks)
ii. Assume a pipeline with 4 stages: fetch instruction (FI), decode instruction and calculate address (DA), fetch operand (FO), and execute (EX). Draw a timing diagram for instruction pipeline operation for a sequence of 9 instructions, in which the fifth instruction is a branch that is taken and in which there is no data dependencies. (3 marks)

b) Describe the structure of SRAM. Give the features of SRAM and DRAM. (2+2 marks)

c) Define virtual address and physical address space. Explain page miss mechanism in virtual memory system. (2+3 marks)

Let me know if you need further assistance!