Previous question solve

20-21:

1(a) Define Computer Organization and Computer Architecture. What is the difference between them? (2 marks)

Computer Organization:
👉 এটা বলে কীভাবে system কাজ করে — অর্থাৎ control signal, data flow, memory access ইত্যাদি নিয়ে কাজ করে।
(“How it operates internally.”)

Computer Architecture:
👉 এটা বলে system কী করে — অর্থাৎ design, instruction set, structure, programmer যা দেখতে পায়।
(“What the system is designed to do.”)

| Aspect | Computer Architecture | Computer Organization |
| --- | --- | --- |
| Focus | Logical design, instruction set | Hardware implementation, circuits |
| Example | Addressing modes, ISA | Control signals, buses |

1(b) What are the key concepts of Von Neumann architecture? Define hardware and software programming. (5 marks)

Von Neumann Architecture Concepts:

- Single memory for both data and instructions.

- Sequential execution — one instruction at a time.

- Stored program concept — program stored in memory.

- ALU, Control Unit, Memory, I/O units are main parts.

- Buses (data, address, control) used to transfer data.

🧠 This is the basic design used in most modern computers.

Hardware Programming:
👉 Writing instructions in machine code or assembly that directly control hardware components.
🔹 Example: Writing MOV A, B in Assembly.

Software Programming:
👉 Writing programs using high-level languages (C, Python, Java) that are later translated into machine code.
🔹 Example: A = B + C; in C language.

1(c) Define Interrupts. What are the approaches for handling multiple interrupts? Illustrate and explain. (5 marks)

Interrupt:
👉 An interrupt is a signal sent to the CPU that temporarily stops the current execution and jumps to an Interrupt Service Routine (ISR) to handle an event.

Example: Keyboard input, Timer, or I/O request.

Approaches for Handling Multiple Interrupts:

- Disabling Interrupts:

  - CPU disables all other interrupts while one is being serviced.

  - Simple but may miss urgent interrupts.

- Fixed Priority:

  - Each interrupt has a fixed priority level.

  - High-priority interrupt can interrupt low-priority ISR.

- Daisy Chaining (Hardware Priority):

  - Devices connected in a chain; the first device has the highest priority.

  - If first device doesn’t request, control passes to next.

- Polling Method:

  - CPU checks each device in sequence to find which requested interrupt.

  - Slower, but simple to implement.

Illustration (Example):

Keyboard Interrupt → Priority 1

Printer Interrupt → Priority 2

Disk Interrupt → Priority 3

If both Keyboard and Disk send interrupt signals,
👉 CPU will handle Keyboard first (highest priority).

1(d) What are the steps of the instruction cycle for the instruction "SUB B, A," which stores the difference of memory locations B and A into location A? (2 marks)

🧩 1(d) Instruction Cycle of “SUB B, A” (2 marks)

Instruction: SUB B, A → A = A - B

Steps:

- Fetch – Memory থেকে instruction নেওয়া।

- Decode – Subtraction identify করা ও operand A, B নির্ধারণ।

- Fetch Operands – A, B এর মান register এ load করা।

- Execute – ALU করে A = A – B।

- Store Result – ফলাফল memory location A তে store করা।

2(a) Define computer instructions and describe their types. (3 marks)

Definition:
👉 A computer instruction is a command given to the CPU to perform a specific task such as data transfer, arithmetic, logic, or control operations.
It is written in machine code or assembly language.

Types of Instructions:

- Data Transfer Instructions: Move data between registers, memory, or I/O.
➤ Example: MOV A, B

- Arithmetic Instructions: Perform mathematical operations.
➤ Example: ADD A, B

- Logical Instructions: Perform logic operations like AND, OR, NOT.
➤ Example: AND A, B

- Control Instructions: Change the sequence of execution (like jump, call).
➤ Example: JMP LABEL

- Input/Output Instructions: Handle data transfer between CPU and I/O devices.
➤ Example: IN, OUT

2(b) Where Zero-address instructions are applicable and how? Explain. (2 marks)

Answer:
👉 Stack-based machine এ use হয়। Operands stack থেকে নেয়।
No explicit operand লাগে না।

Example:

PUSH A

PUSH B

ADD   ← top দুই value add করে

Used in stack computers like JVM, HP calculators.

2(c) What are logical, arithmetic, and cyclic (rotate) shifts? Apply a 4-bit logical right shift, logical left shift, arithmetic right shift, arithmetic left shift, right rotate, left rotate to the bits 10100110. (5 marks)

Definition:

🧠 1️⃣ Logical Shift

👉 Logical shift মানে হলো — বিটগুলোকে ডান বা বামে সরানো, আর ফাঁকা জায়গাগুলোতে 0 বসানো।
🔹 Sign bit (MSB)-এর কোনো মানে এখানে থাকে না, কারণ এটা শুধু bit shifting-এর জন্য ব্যবহৃত হয়।

⚙️ 2️⃣ Arithmetic Shift

👉 Arithmetic shift ব্যবহৃত হয় signed numbers (two’s complement) এর ক্ষেত্রে।
এখানে sign bit (MSB) ধরে রাখা হয় যেন সংখ্যা positive/negative ঠিক থাকে।

🔄 3️⃣ Cyclic (Rotate) Shift

👉 Rotate মানে হলো শেষ দিকের bit আবার শুরুতে ফিরে আসে (বা উল্টোভাবে)।
কোনো bit হারায় না, শুধু ঘুরে যায়।

আমাদের মূল 8-bit সংখ্যা:

10100110

🧩 1️⃣ 4-bit Logical Right Shift (>>4)

👉 সব বিট ৪ ঘর ডানে সরবে, বাম পাশে ০ বসবে।

Original: 10100110

Shifted : 00001010

✅ Result: 00001010

🧩 2️⃣ 4-bit Logical Left Shift (<<4)

👉 সব বিট ৪ ঘর বামে সরবে, ডান পাশে ০ বসবে।

Original: 10100110

Shifted : 01100000

✅ Result: 01100000

🧩 3️⃣ 4-bit Arithmetic Right Shift

👉 Sign bit (MSB) অপরিবর্তিত থাকবে, বাকি বিট ৪ ঘর ডানে সরবে।
(বাম পাশে sign bit পূরণ হবে)

Original: 10100110

Sign bit: 1

Shifted : 11111010

✅ Result: 11111010

🧩 4️⃣ 4-bit Arithmetic Left Shift

👉 প্রায় Logical Left Shift এর মতোই — শুধু overflow হলে sign বদলাতে পারে।
এখানে বামে ৪ ঘর সরিয়ে ডান পাশে ০ বসাই।

Original: 10100110

Shifted : 01100000

✅ Result: 01100000

🌀 5️⃣ 4-bit Right Rotate

👉 শেষের ৪ বিট প্রথমে চলে আসবে, প্রথমের ৪ বিট শেষে যাবে।

Original: 1010 | 0110

Rotate Right 4 → 0110 | 1010

✅ Result: 01101010

🌀 6️⃣ 4-bit Left Rotate

👉 প্রথমের ৪ বিট শেষে চলে যাবে, শেষের ৪ বিট প্রথমে আসবে।

Original: 1010 | 0110

Rotate Left 4 → 0110 | 1010

✅ Result: 01101010
(দেখো, 8-bit সংখ্যায় 4-bit rotate left = rotate right একই ফল দেয়)

✅ Final Summary Table

| Operation | Result (8-bit) |
| --- | --- |
| Logical Right Shift (4-bit) | 00001010 |
| Logical Left Shift (4-bit) | 01100000 |
| Arithmetic Right Shift (4-bit) | 11111010 |
| Arithmetic Left Shift (4-bit) | 01100000 |
| Right Rotate (4-bit) | 01101010 |
| Left Rotate (4-bit) | 01101010 |

2(d) What is a procedure, and why is it necessary? Illustrate and explain the procedure mechanism. (4 marks)

Definition:
👉 A procedure (or subroutine/function) is a block of code that performs a specific task and can be called from anywhere in the program.

Necessity / Advantages:

- Code Reusability: One procedure can be used many times.

- Modularity: Breaks a big program into smaller parts.

- Easy Debugging and Maintenance.

Procedure Mechanism:

- Call: CPU saves the return address (next instruction) in the stack and jumps to the procedure.

- Execute: Procedure instructions run.

- Return: Control comes back to the saved return address.

Example:

CALL ADDITION    ; call procedure

...

ADDITION:         ; procedure starts

ADD A, B

RET            ; return to main program

3(a) What is an I/O module, and why is it necessary? (3 marks)

Definition:

I/O Module কী?

👉 এটি একটা hardware অংশ যা CPU এবং বাইরে থাকা devices (যেমন keyboard, mouse, printer) এর মধ্যে যোগাযোগ করায়।

কারণে লাগে / কাজগুলো:

- Device Communication:
CPU এবং external devices-এর মধ্যে data path তৈরি করে।
যেমন: Keyboard থেকে typed key CPU পর্যন্ত পৌঁছানো।

- Data Conversion:
Device-এ signal হতে পারে analog বা serial, CPU binary বোঝে।
I/O module এটাকে পরিবর্তন করে CPU-compatible করে।

- Control / Management:
Device ready কি না, error আছে কি না, timing ঠিক আছে কি না—সব দেখাশোনা করে।

- CPU Load কমায়:
Slow device direct handle করলে CPU wait করতে হবে। I/O module handle করলে CPU অন্য কাজ করতে পারে।

Example:

- তুমি keyboard-এ key চাপছো।

- I/O module সেটা input নেয় → binary তে convert করে → CPU-কে দেয়।

3(b) What are the differences among Programmed I/O, Interrupt-driven I/O, and Direct Memory Access (DMA)? Describe with flowcharts. (5 marks)

🧠 1️⃣ Programmed I/O (Polling method)

👉 এখানে CPU নিজেই বারবার দেখে ডিভাইস (যেমন কীবোর্ড, প্রিন্টার) রেডি কিনা।
ডিভাইস রেডি না থাকলে CPU অপেক্ষা করে — কিছুই করতে পারে না।

📖 Example:
তুমি কারো ফোন আসার অপেক্ষায় ফোনটা তাকিয়ে তাকিয়ে দেখছ — অন্য কিছু করছ না।

🔹 Process:

- CPU → I/O device-কে বলে: "তুমি data পাঠাও।"

- CPU বারবার চেক করে: "রেডি হয়েছো?"

- রেডি হলে CPU ডেটা নেয়।

- তারপর পরের কাজ করে।

🕒 Problem: CPU সময় নষ্ট করে (busy waiting)।

⚡ 2️⃣ Interrupt-driven I/O

👉 এখানে ডিভাইস রেডি হলে নিজেই CPU-কে interrupt পাঠায়।
ততক্ষণ CPU অন্য কাজ করতে পারে।

📖 Example:
তুমি ফোনটা রেখে নিজের কাজ করছ। ফোন বাজলেই (interrupt) তুমি ধরো।

🔹 Process:

- CPU I/O-কে বলে: “তুমি রেডি হলে ডাক দিও।”

- CPU নিজের অন্য কাজ করে।

- ডিভাইস রেডি হলে interrupt পাঠায়।

- CPU কাজ থামিয়ে I/O হ্যান্ডেল করে, তারপর আগের কাজ চালায়।

✅ Benefit: CPU সময় বাঁচায়, parallel কাজ হয়।

🚀 3️⃣ Direct Memory Access (DMA)

👉 এখানে ডেটা I/O ডিভাইস ↔ মেমোরির মধ্যে সরাসরি ট্রান্সফার হয়, CPU-কে পাশ কাটিয়ে।

📖 Example:
তুমি বন্ধুকে বললে, “এই ফাইলটা কপি করে দে” — তুমি নিজে না করে বন্ধু (DMA controller) করে দিল।

🔹 Process:

- CPU শুধু DMA controller-কে বলে: “এই ডেটা ওখানে পাঠাও।”

- DMA নিজে ডেটা ট্রান্সফার করে।

- কাজ শেষ হলে CPU-কে interrupt পাঠায়।

✅ Benefit: CPU ফ্রি থাকে, দ্রুত ডেটা ট্রান্সফার হয়।
📈 Use: বড় ডেটা ট্রান্সফারে যেমন — হার্ডডিস্ক, মেমরি কপি ইত্যাদি।

📊 Comparison Table

| Feature | Programmed I/O | Interrupt-driven I/O | DMA |
| --- | --- | --- | --- |
| Who controls transfer? | CPU | CPU (with interrupt) | DMA Controller |
| CPU waiting? | Always busy | Not busy | Not busy |
| Speed | Slow | Faster | Fastest |
| Use case | Small devices | Medium-speed devices | High-speed data transfer |
| CPU load | High | Medium | Very Low |

🧩 সহজ মনে রাখার উপায়:

| ধরো তুমি ডেলিভারি দিতে বললে | উদাহরণ |
| --- | --- |
| তুমি নিজেই হাতে হাতে দিচ্ছো | Programmed I/O |
| ডেলিভারি বয় রেডি হলে ফোন দিচ্ছে | Interrupt-driven I/O |
| ডেলিভারি বয় নিজেই দিয়ে শেষে জানাচ্ছে | DMA |

3(c) How does a DMA module transfer control to the processor after its task? Does it interrupt the processor? Define cycle stealing in DMA. (2 marks)

Answer:
✅ DMA কাজ শেষ হলে CPU কে interrupt পাঠায় জানাতে যে কাজ শেষ।
👉 So yes, DMA interrupts the processor.

Cycle Stealing:
DMA এক memory cycle এর জন্য CPU থেকে bus নিয়ে নেয় data transfer এর জন্য।
CPU সাময়িক pause হয় 1 cycle এর জন্য — পরে আবার কাজ চালায়।

একদম সহজভাবে বুঝি 👇

💡 Cycle Stealing (in DMA) মানে কী?

👉 যখন DMA controller ডেটা ট্রান্সফার করে, তখন তার memory access করতে হয় —
কিন্তু memory bus তো CPU-ও ব্যবহার করে!

তাই DMA বলে —
“CPU ভাই, একটু সময়ের জন্য bus টা দাও, আমি শুধু ১টা memory cycle ব্যবহার করবো।” 😄

⚙️ What happens:

- CPU কাজ করছে 🧠

- DMA এক মুহূর্তের জন্য CPU থেকে bus “চুরি” নেয় (steal করে)।

- ঐ সময় DMA ডেটা ট্রান্সফার করে (I/O ↔ Memory)।

- ট্রান্সফার শেষে DMA bus ফিরিয়ে দেয় CPU-কে।

- CPU আবার আগের কাজ চালায়।

🕒 Result:

- CPU এক cycle এর জন্য pause হয়, পুরোপুরি বন্ধ হয় না।

- তাই এটাকে বলা হয় Cycle Stealing — কারণ DMA “একটা cycle” ধার নেয়।

📖 Example দিয়ে বুঝো:

ধরো তুমি (CPU) ক্লাসে লিখছ, হঠাৎ বন্ধু (DMA) বলে:

“এক সেকেন্ড কলমটা দে, একটা শব্দ লিখে দিই!”

তুমি ১ সেকেন্ড থামলে, সে লিখে কলম ফেরত দিল।
তুমি আবার লিখা চালিয়ে গেলে।
👉 এটাই হলো Cycle Stealing।

3(d) Give the features of CISC and Core i7 in the perspective of computer architecture and organization. (4 marks)

CISC (Complex Instruction Set Computer):

- অনেক complex instruction

- এক instruction এ multiple task

- Assembly-friendly

- Slow execution but short code
Example: Intel 8086, Pentium

Core i7 (Modern Intel Processor):

- CISC ভিত্তিক কিন্তু ভিতরে RISC micro-ops

- Multi-core + Hyper-threading

- L1/L2/L3 Cache

- Branch prediction, pipelining, GPU integrated
🧠 Result → High performance modern processor.

অবশ্যই! 😄 চলো একদম সহজভাবে CISC vs Core i7 তুলনা করি computer architecture & organization এর দিক থেকে 👇

1️⃣ CISC (Complex Instruction Set Computer)

- Instruction set: অনেকগুলো জটিল এবং বড় কমান্ড থাকে।
→ একেকটা instruction অনেক কাজ করতে পারে।

- Programming: assembly প্রোগ্রাম করা সহজ।

- Execution: এক instruction-এ অনেক operation → কিন্তু একেকটা instruction ধীর।

- Code size: ছোট কোড, কারণ এক instruction অনেক কাজ করে।

- Example: Intel 8086, Pentium (পুরনো processors)

সারসংক্ষেপ:

CISC = “Instruction-heavy, slower execution, compact code”

2️⃣ Core i7 (Modern Intel processor)

- Based on CISC internally: কিন্তু ভিতরে instruction কে RISC-like micro-operations এ ভাগ করে execute করে।

- Performance:

  - Multi-core → একসাথে অনেক কাজ করতে পারে

  - Hyper-threading → একটি core multiple threads handle করে

  - Pipelining → এক instruction চালাতে গেলে next instruction শুরু হয়ে যায়

- Cache: L1, L2, L3 cache → দ্রুত ডেটা access

- Advanced features: branch prediction, out-of-order execution, integrated GPU

সারসংক্ষেপ:

Core i7 = “Modern CISC with RISC-like execution, high speed, multi-core, advanced optimizations”

✅ Comparison Table (Simple Version)

| Feature | CISC | Core i7 |
| --- | --- | --- |
| Instruction Set | Complex, many operations | CISC based, internally uses RISC ops |
| Execution | Slower per instruction | Very fast (pipelining, out-of-order) |
| Code size | Compact | Medium (depends on micro-ops) |
| CPU Features | Basic | Multi-core, Hyper-threading, Cache, Branch prediction |
| Example | Intel 8086, Pentium | Intel Core i7 |

💡 Remember:
CISC = পুরনো traditional design
Core i7 = আধুনিক processor, CISC এর evolution + speed optimization

4(a)

Step 0: Given Opcodes

| Operation | Opcode |
| --- | --- |
| Load from Memory | 1110 |
| Store to Memory | 0001 |
| Add | 0101 |
| Subtract | 1011 |
| Store to I/O | 0110 |
| Load from I/O | 1101 |

Program:
1️⃣ Load AC from device 99
2️⃣ Add contents of memory 555
3️⃣ Store AC to memory 666

Step 1: Convert addresses to 12-bit binary & hex

| Address | 12-bit binary | Hex |
| --- | --- | --- |
| 99 | 000001100011 | 063 |
| 555 | 001000101011 | 22B |
| 666 | 001010011010 | 29A |

Step 2: Combine Opcode + Address → 16-bit instruction

| Step | Instruction (Binary) | Hex | Meaning |
| --- | --- | --- | --- |
| 1 | 1101 000001100011 | D063 | Load AC from I/O 99 |
| 2 | 0101 001000101011 | 522B | Add memory[555] to AC |
| 3 | 0001 001010011010 | 129A | Store AC to memory 666 |

Step 3: Execution (Fetch → Decode → Execute)

1️⃣ Load AC from I/O 99 (D063)

- Fetch: CPU reads instruction D063 from memory.

- Decode: Opcode 1101 → Load from I/O; Address = 99.

- Execute: Read data from device 99 → AC = device[99]

2️⃣ Add memory[555] (522B)

- Fetch: CPU reads 522B.

- Decode: Opcode 0101 → ADD; Address = 555

- Execute: Read memory[555] → add to AC → AC = AC + memory[555]

3️⃣ Store AC to memory[666] (129A)

- Fetch: CPU reads 129A.

- Decode: Opcode 0001 → STORE; Address = 666

- Execute: AC → memory[666] → memory[666] now stores AC value

Easy way to remember:

- Step 1: Load → AC gets value from device

- Step 2: Add → AC + memory

- Step 3: Store → AC → memory

✅ AC always holds the “current result” during execution.

4(b) (2 + 2 marks)

(i) Explain RAID-2 (সংক্ষিপ্ত):

- RAID-2 হল ঐতিহাসিক/শিক্ষণধর্মী RAID level যেখানে bit-level striping ব্যবহার করা হয়।

- প্রতিটি ডাটা-বাইট (বা শব্দ) প্রতিটি ডিস্কে bit-wise ছড়িয়ে দেওয়া হয় এবং dedicated Hamming-code parity/disks ব্যবহার করে ভুল(s) সনাক্ত ও ঠিক করা হয় (error-correcting code)।

- প্রাকৃতিকভাবে, RAID-2-এ দরকার হয়ে পড়ে অনেক ডিস্ক (একাধিক parity/disks for ECC)।

- বাস্তবে RAID-2 খুবই কম ব্যবহৃত কারণ Hamming-ECC টেকনিকগুলো পরে ডিস্ক-কন্ট্রোলার স্তরে করা সহজে এবং আরেকটু দক্ষ RAID schemes (RAID-5 ইত্যাদি) জনপ্রিয় হয়েছে।

(ii) Semiconductor memory-এর ৪টি প্রধান ধরন ও তাদের বৈশিষ্ট্য (সংক্ষিপ্ত):

- SRAM (Static RAM)

  - গঠন: flip-flop-based (6-transistor cell)।

  - বৈশিষ্ট্য: দ্রুত (high speed), volatile (power off → ডেটা যায়), দাম বেশি, ব্যবহার: CPU caches (L1, L2)।

- DRAM (Dynamic RAM)

  - গঠন: ক্যাপাসিটর + transistor (1T-1C)।

  - বৈশিষ্ট্য: সস্তা, বেশি ঘনত্ব (high density), ধীরতর তুলনায় SRAM, volatile, periodical refresh প্রয়োজন। ব্যবহার: main memory (RAM)।

- ROM (Read Only Memory) / Mask ROM

  - গঠন: প্রি-programmed memory; power off-এ ডেটা থাকে (non-volatile)।

  - বৈশিষ্ট্য: read-only বা limited programmability; ব্যবহার: firmware, bootstrap code।

- EPROM / EEPROM / Flash (Electrically Programmable ROM)

  - গঠন: floating-gate transistor cell।

  - বৈশিষ্ট্য: non-volatile, electrically erasable/reprogrammable (EEPROM/Flash), slower write/erase than SRAM/DRAM, ব্যবহার: firmware storage, SSD chips (NAND flash) ইত্যাদি।

4(c)

Important Addressing Modes (Easy Version)

- Immediate Addressing

- Operand instruction-এর ভিতরই থাকে (constant value)

- Example: MOV A, #5 → A তে 5 বসানো

- Direct Addressing

- Instruction-এ ঠিক memory address দেওয়া থাকে

- Example: MOV A, 1000 → memory[1000] থেকে value → A

- Indirect Addressing

- Instruction-এ এমন address থাকে যেটা বলে memory-তে কোথায় operand আছে

- Example: address 2000 → memory[2000] = 1000 → memory[1000] = actual value

- Register Addressing

- Operand CPU register-এ থাকে

- Example: ADD A, B → A + B (B register থেকে)

- Indexed Addressing

- Base address + index register → effective address

- Example: MOV A, 2000 + X → array element access

- Stack Addressing (Implicit)

- Operand stack top থেকে আসে

- Example: PUSH বা POP

Memory trick to remember:

- Immediate: value already in instruction

- Direct: memory address directly

- Indirect: memory of memory

- Register: CPU register

- Indexed: array / offset

- Stack: LIFO

🧩 4(c)(ii) Two’s Complement Operations

(a) -A + B (Example)

Let’s say A = 5, B = 18
A = 000101, B = 010010
-A = invert +1 → 111011
Now:

111011

+010010

=1001101 → discard carry → 001101 = 0x0D

✅ Result = +13 (Hex 0x0D)

(b) -33 - 17

Range of 6-bit two’s complement = -32 … +31
→ -33 not representable ⇒ Overflow ❌

So result not possible within 6-bit limit.

5(a) Cache Mapping

Given:

- Byte addressable memory: 2¹⁶ = 65,536 bytes

- Block size = 8 bytes

- Direct mapped cache, 32 lines

Step i: Divide 16-bit memory address

Formulas:

- Byte number (within block): log2(block size) = log2(8) = 3 bits

- Line number (cache line): log2(number of lines) = log2(32) = 5 bits

- Tag: Remaining bits = 16 – (5+3) = 8 bits

✅ Summary:

| Field | Bits | Meaning |
| --- | --- | --- |
| Tag | 8 | Identify block uniquely |
| Line Number | 5 | Which cache line |
| Byte Number | 3 | Which byte in block |

Step ii: Cache line for given addresses

- Address: 0001 0001 0001 1011

Step 1: Divide into tag-line-byte

- Byte number = last 3 bits → 011 = 3

- Line number = next 5 bits → 00011 = 3

- Tag = remaining 8 bits → 00010001

→ Cache line = 3

- Address: 1101 0000 0001 1101

- Byte number = last 3 bits → 101 = 5

- Line number = next 5 bits → 00011 = 3

- Tag = remaining 8 bits → 11010000

→ Cache line = 3

দুটো address একই cache line-এ map হবে, তবে tag ভিন্ন → conflict miss হতে পারে

Step iii: Other bytes in the block

- Address: 0001 1010 0001 1010

- Block size = 8 bytes → block-aligned addresses = last 3 bits = 0–7

- Block start address: clear last 3 bits → 0001 1010 0001 1000

- Block bytes:

0001 1010 0001 1000

0001 1010 0001 1001

0001 1010 0001 1010  ← given

0001 1010 0001 1011

0001 1010 0001 1100

0001 1010 0001 1101

0001 1010 0001 1110

0001 1010 0001 1111

Step iv: Total bytes cache can store

- Cache lines = 32

- Each line holds 1 block = 8 bytes

- Total cache size = 32 × 8 = 256 bytes

5(b) Booth's Multiplication Algorithm

Given:

- Multiplicand ( Q = -17 )

- Multiplier ( M = 16 )

- Use 8-bit representation for Q and M

Step 1: Convert to binary (2’s complement)

- Q = -17 → 8-bit two’s complement

- +17 = 00010001

- Invert bits: 11101110

- Add 1: 11101111 ✅ → Q = 11101111

- M = 16 → binary 8-bit → 00010000

Step 2: Initialize registers

| Register | Initial Value |
| --- | --- |
| A | 00000000 |
| Q | 11101111 |
| Q-1 | 0 |
| Count | 8 |

Step 3: Booth’s Algorithm Rules

- Check Q0 Q-1:

- 01 → A = A + M

- 10 → A = A – M

- 00 or 11 → do nothing

- After operation → Right Arithmetic Shift of [A Q Q-1]

Step 4: Stepwise Execution (8 cycles)

Notation: [A | Q | Q-1]

Initial:

A = 00000000
Q = 11101111
Q-1 = 0
[00000000 11101111 0]

Cycle 1:

- Q0 Q-1 = 1 0 → A = A – M = 0 – 16 = -16 → 11110000 (8-bit)

- Right arithmetic shift → [11111000 01110111 1]

Cycle 2:

- Q0 Q-1 = 1 1 → do nothing

- Right arithmetic shift → [11111100 00111011 1]

Cycle 3:

- Q0 Q-1 = 1 1 → do nothing

- Right arithmetic shift → [11111110 00011101 1]

Cycle 4:

- Q0 Q-1 = 1 1 → do nothing

- Right arithmetic shift → [11111111 00001110 1]

Cycle 5:

- Q0 Q-1 = 0 1 → A = A + M = 11111111 + 00010000 = 00001111 (overflow ignored, 8-bit)

- Right arithmetic shift → [00000111 10000111 0]

Cycle 6:

- Q0 Q-1 = 1 0 → A = A – M = 00000111 – 00010000 = 11110111

- Right arithmetic shift → [11111011 11000011 1]

Cycle 7:

- Q0 Q-1 = 1 1 → do nothing

- Right arithmetic shift → [11111101 11100001 1]

Cycle 8:

- Q0 Q-1 = 1 1 → do nothing

- Right arithmetic shift → [11111110 11110000 1]

✅ Step 5: Final Result

- [A Q] = 11111110 11110000 → 16-bit signed

- Decimal = -272 ✅

Shortcut / Quick Check:

- Q = -17, M = 16 → -17 × 16 = -272 ✅

Exam Tip:

- Draw A, Q, Q-1 table

- Show operation (+M or –M) at each step

- Show arithmetic right shift clearly

- Final [A Q] → product

5(c)(i) Access Time & Cycle Time

- Access Time

- Time to read/write a word from memory once

- Example: 60 ns

- Cycle Time

- Minimum time between starting two successive memory operations

- Includes access + recovery time

Access time ≤ Cycle time

5(c)(ii) Memory Access Types

| Type | Explanation / Example |
| --- | --- |
| Sequential | Memory accessed one after another in order (tape). Slow. |
| Direct | Memory can jump directly to a block without going sequentially (CD). |
| Random | Any word can be accessed directly using address (RAM). Fastest. |

✅ Quick Summary / Exam Tip:

- Cache → divide address: Tag | Line | Byte

- Total cache bytes = lines × block size

- Booth → check Q0 Q-1 bits, shift & add/sub

- Memory access → sequential/direct/random

ঠিক আছে, চলো Question 6 step by step সহজভাবে solve করি, বাংলা–English mix এ। 😄

6(a)(i) Responsibility of Memory Management Unit (MMU)

Definition:
👉 MMU হল CPU-এর অংশ যা virtual memory এবং physical memory-এর মধ্যে mapping করে।

Responsibilities / কাজগুলো:

- Address Translation:

- Virtual address (program-এ ব্যবহৃত) → Physical address (RAM location)

- CPU শুধুমাত্র virtual address generate করে, MMU সেটা physical address-এ convert করে।

- Memory Protection:

- Program একাধিক memory region-এ access করতে পারে, MMU নিশ্চিত করে illegal access না হয়।

- Example: User program memory overflow বা OS memory access control।

- Paging / Segmentation Support:

- Virtual memory management এর জন্য page table / segment table maintain করে।

- Cache Control (Optional):

- কিছু MMU cacheable / non-cacheable memory region control করে।

6(a)(ii) Instruction Pipeline Timing Diagram

Given:

- Pipeline stages:

  - FI = Fetch Instruction

  - DA = Decode Instruction & calculate address

  - FO = Fetch Operand

  - EX = Execute

- 9 instructions (I1 to I9)

- I5 = branch taken

- No data dependencies

Timing Diagram (simplified):

| Cycle → | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I1 | FI | DA | FO | EX |  |  |  |  |  |  |  |  |
| I2 |  | FI | DA | FO | EX |  |  |  |  |  |  |  |
| I3 |  |  | FI | DA | FO | EX |  |  |  |  |  |  |
| I4 |  |  |  | FI | DA | FO | EX |  |  |  |  |  |
| I5 (branch) |  |  |  |  | FI | DA | FO | EX |  |  |  |  |
| I6 |  |  |  |  |  | FI | DA | FO | EX |  |  |  |
| I7 |  |  |  |  |  |  | FI | DA | FO | EX |  |  |
| I8 |  |  |  |  |  |  |  | FI | DA | FO | EX |  |
| I9 |  |  |  |  |  |  |  |  | FI | DA | FO | EX |

💡 Note: Branch taken → I6 starts from the branch target after EX of I5. No stalls due to no data dependency.

6(b) Structure of SRAM and Features

SRAM Structure:

- Static RAM uses flip-flop circuits to store each bit.

- Typical cell = 6 transistors (no capacitor needed).

- Stored bit stays as long as power on।

Features of SRAM:

- Fast (high speed)

- Volatile (power off → data lost)

- Expensive per bit

- Used in CPU cache (L1, L2)

Features of DRAM:

- Dynamic RAM uses 1 transistor + 1 capacitor per bit

- Slower than SRAM

- Needs periodic refresh

- Cheaper and high-density → Used as main memory

6(c) Virtual & Physical Address, Page Miss

Definitions:

- Virtual Address: CPU-generated address by program, may not directly map to RAM.

- Physical Address: Actual address in main memory (RAM).

Page Miss Mechanism (Virtual Memory):

- CPU requests virtual address → MMU translates to physical address

- If page not in main memory → Page Miss / Page Fault occurs

- OS interrupts CPU → loads the required page from secondary storage (disk) to RAM

- Page table updated → CPU retries instruction

💡 Easy Example:

- Program wants byte 0x1F234 (virtual)

- Not in RAM → page miss → OS brings page 0x1F000–0x1F3FF into RAM → instruction executes